OXFORD PLANETARY GROUP

TITLE: 	 		RADTRANS

AUTHORS:

..........................................

P.G.J. IRWIN

DOCUMENT NO.:	NIMS/OX/PGJI/SW/136

DOCUMENT TITLE

RADTRANS

DOCUMENT NUMBER: NIMS/OX/PGJI/SW/136

| DOCUMENT STATUS SHEET |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISSUE |  | REV |  | DATE |  |  | REASON FOR CHANGE |  |  |
| 1 |  | 0 |  | 12/11/94 |  |  | New Software Release |  |  |
| 2 |  | 0 |  | 19/12/94 |  |  | Addition of Scattering |  |  |
| 2 |  | 1 |  | 20/3/95 |  |  | Addition of further features |  |  |
| 2 |  | 2 |  | 30/7/96 |  |  | Major text revisions |  |  |
| 2 |  | 3 |  | 23/10/98 |  |  | Long overdue revision |  |  |
| 2 |  | 3a |  | 13/1/00 |  |  | Minor corrections |  |  |
| 3 |  | 0 |  | 1/7/04 |  |  | Long overdue overhaul |  |  |
| 3 |  | 1 |  | 5/10/09 |  |  | Another long due overhaul to update the location of various directories. |  |  |
| 3 |  | 2 |  | 30/4/12 |  |  | Another overhaul to link with svn control |  |  |
| DISTRIBUTION LIST |  |  |  |  |  |  |  |  |  |
| NAME |  |  |  |  | NAME |  |  |  |  |
| 1 | P. Irwin |  | Oxford |  |  |  |  |  |  |
| 1 | N. Teanby |  | Oxford |  |  |  |  |  |  |
| 1 | N. Bowles |  | Oxford |  |  |  |  |  |  |
| 1 | C. Wilson |  | Oxford |  |  |  |  |  |  |

Table of Contents

0. Overview	1

0.1 Reference Documents	1

0.2 Defined fonts	2

1. Introduction	3

1.1 Location of code and reference files	5

2. Atmospheric T,P and VMR Profile Files (.prf)	7

2.1 Profile	8

3. Dust/Aerosol Profiles	11

4. Path	13

4.1 Layers	13

4.2 Paths	15

4.3 Calculation Types	16

4.4 Path File Format	17

4.5 Running Path	24

4.6 Changes in driver file format from Genlbl	25

4.7 Non-LTE	25

5. Radtrans	26

5.1 Line by Line Models (ICONV<10)	27

5.1.2 Line handling in Radtrans	28

5.2 Band Models 9<ICONV<20	28

5.2.1 Band Models using Line data (ICONV=10,11)	28

5.2.2 Band Models using Band data (11<ICONV<20)	30

5.2.3 Summary of Radtrans Band Models	32

5.3 Correlated-K models 19<ICONV<30	32

5.3.1 Correlated-K model using line data (ICONV=20)	34

5.3.2 Malkmus-Lorentz Band Correlated-K (ICONV=21)	34

5.3.3 Goody-Voigt Band Correlated-k (ICONV=22)	36

5.3.4 Exponential Sum Models (ICONV=23)	37

5.3.5 k-Distribution Look-up Table (ICONV=24)	37

5.4 Running Radtrans	37

6. Post Processing of Radtrans Output Spectra	39

6.1 Conv_spec	39

6.2 Li_spec	40

6.3 Pl_spec	40

6.4 Aground	44

6.5 Diskav	44

7. Setting up Spectral Data	45

7.1 Line data	45

7.1.1 Makedb	45

7.1.2 Scan	46

7.1.3 Check_index	46

7.1.4 Li_lines	46

7.1.5 Pl_lines	46

7.1.6 Select	46

7.1.7 Selecttemp	48

7.1.8 Selecttemploop	49

7.1.9 Cp_lines	49

7.1.10 Summary	49

7.1.11 Merge	49

7.1.12 Line data Strengths and Absorber Amounts Units	49

7.1.13 Partition Functions	50

7.2 Band Data	51

7.2.1 Makeband	52

7.2.2 Convert_data	54

7.3 Correlated-k data	54

7.4 Gas Continua	54

8. Scattering	56

8.1 Phase Functions and Normalisation	56

8.1.1 Phase functions used in Radtrans	57

8.2 Fourier Decomposition	57

8.3 Doubling and adding	58

8.3.1 Quadrature details	60

8.4 Miscellaneous Calculation Details	61

8.5 Calculation scattering properties	63

8.5.1 Calculating scattering properties with Makephase	63

8.5.2 Format of ‘.sca’ file	65

Appendix A Radtrans Gas ID Codes	68

# 0. Overview

This document describes the radiative transfer model Radtrans which has been developed over a number of years by the Planetary Group in Oxford for the modelling of radiances in planetary atmospheres.

## 0.1. Reference Documents

[R1]	Atmospheric Radiation. Theoretical Basis 2nd Ed, R.M. Goody and Y.L.Yung (1989). Oxford University Press

[R2]	Integrated Absorption of a Spectral Line with the Voigt Profile, C.D.Rodgers and A.P.Williams (1974). J.Quant. Spectrosc. Radiat. Transfer 14 pp 319-323

[R3]	Approximate Methods of Calculating Transmission by Bands of Spectral Lines, C.D.Rodgers (1976). NCAR/TN-116+1A

[R4]	Spectral Parameters of Methane for Remote Sounding of the Jovian Atmosphere, E.K.Strong (1992). D.Phil.Thesis, Oxford

[R5]	AFCRL Atmospheric Absorption Line Parameters Compilation, McClatchey et al (1973). AFCRL-TR-73-0096

[R6]	A Description of the Correlated k Distribution Method for Modelling Nongray Gaseous Absorption, Thermal Emission, and Multiple Scattering in Vertically Inhomogeneous Atmospheres, A.A.Lacis and V.Oinas (1991). J.G.R. 96 pp 9027-9063

[R7]	The Correlated-k Method for Radiation Calculations in Nonhomogeneous Atmospheres, R.Goody, R.West, L.Chen and D.Crisp (1989), J.Quant. Spectrosc. Radiat. Trans. 42 pp 539-550

[R8]	Calculated Tabulations of H2O Line Broadening by H2O, N2, O2 and CO2 at High Temperature, C.Delaye, J-M Hartmann and J.Taine (1989), Appl. Opt. 28 pp 5080

[R9]	Estimation of Rotational Line Widths of Carbon Dioxide Bands, G.Yamamoto, M.Tanaka and T.Aoki (1969), J.Quant. Spectrosc. Radiat. Trans. 9 pp 371-382

[R10]	Oxford Planetary Group Radiative Transfer Models, P. Irwin (1994), NIMS/OX/PGJI/SW/133.

[R11]	Light Scattering in Planetary Atmospheres, J.E.Hansen and L.D.Travis (1974), Space Sci. Rev 16 527-610.

[R12]	Modelling of Pressure-Induced FIR Absorption spectra: Molecular Hydrogen Pairs, J.Borysow et al. (1985), Astrophys.J. 296 pp 644-654

[R13]	Collision-induced Rototranslational Absorption Spectra of H2_He pairs at temperatures from 40 to 3000K, J.Borysow et al. (1988), Astrophys.J. 326 pp 509-515

[R14]	Nemesis, P. Irwin (2004), CIRS/OX/TR/1392.

[R15]	Giant Planets of our Solar System, P.Irwin (2003), Springer-Praxis.

[R16]	Radtran Scattering Code, P. Irwin (1997) NIMS/OX/PGJI/SW/267

[R17]	NIMS Scattering Study, P. Irwin (1997), NIMS/OX/PGJI/SW/268.

[R18]	Multiple scattering of polarized light in planetary atmospheres. Part II. Sunlight reflected by terrestrial water clouds, Hansen, J. E (1971), J. Atm. Sci., 28, pp 1400-1426,

[R19]	Matrix operator theory of radiative transfer. 1: Rayleigh scattering, G.N. Plass et al. (1973), Appl. Opt., 12, pp 314- 329

## 0.2. Defined Fonts

As in [R14], in an attempt to make this document easier to read the following fonts are used to denote different objects.

- Executable programs are underlined. e.g. Radtrans

- Suites of codes in their own subdirectories are in copperplate font. e.g. Radtrans

- Subroutine files are in courier font. e.g. genrads.f

- Variables defined within FORTRAN codes are capitalized. e.g. NCONV, IMOD.

# 1. Introduction

The Radtrans model calculates the transmission, absorption or thermal emission spectra of gaseous paths using one of three radiative transfer (RT) models: 1) line-by-line, 2) band or 3) correlated-k [R1]. In addition to transmission, absorption or thermal emission, Radtrans may compute multiple-scattered spectra, and hemispherically-averaged spectra. Depending on the RT model chosen, spectral information is read in either from line databases, averaged band parameter, or k-distribution files. All the sub-models use identical methods for calculating the atmospheric gas paths and dust opacities and have identical outputs to allow easy intercomparison. The atmospheric profiles are read in from ‘.prf’ files which contain all the information required to model the atmosphere of any planet. The models are thus not limited to the Earth’s atmosphere like some but are intended to be general and simple to set up for any planet.

Radtrans is a development from Genlbl, a general purpose line-by-line transmission calculation routine written by Simon Calcutt. Radtrans combines the LBL functions of Genlbl with faster, though less accurate, band and correlated-k models described in [R10]. The model takes as its input a ‘.drv’ driver file created by the program Path (which takes as its input a ‘.pat’ path definition file) and reads in the required spectral data files together with any additional files required by the particular RT model used (for example dust optical properties and g-intervals). An overview of the processing process is shown in Figure 1.1.

Path file

(.pat)

Driver file

(.pat)

Key file

(.key)

Path

Cross-section file

(.xsc)

g-interval file

(.gin)

Scattering file

(.sca)

(Different format for LBL, band or c-k models)

(correlated-k model only)

(scattering model only)

Radtrans

Binary output file

(.out)

Conv_spec

Binary averaged output file (.ave)

Pl_spec

Li_spec

Output ASCII files

Fig. 1.1 Overview of Radtrans Processing Software

Spectra are calculated in wavenumber space only and the calculated spectra are written to binary ‘.out’ files. Hence these output files must be processed using one of the three post-processors Conv_spec, Li_spec and Pl_spec which are described later and which may transform the spectra to wavelength space if required and write the final output spectra in ASCII form.

The basic procedure for calculating a spectrum is this:

- Construct the ‘.pat’ path file which defines which atmospheric profiles to use, which spectral information to include and the calculation type required.

- Run Path to create the ‘.drv’ driver file, for which the atmospheric layer properties (pressure, temperature, absorber amount etc.) have been calculated.

- Run Radtrans to compute the output binary spectra.

- Process the raw binary ‘.out’ files to the required output using either Conv_spec, Li_spec or Pl_spec (or in some cases Aground, Diskav)

This document is laid out in same the order of these steps, except that a description of the atmospheric ‘.prf’ files is first described. Please note that reference ‘.prf’ files and also spectral databases are already available in the directories listed in section 1.1. New users of Radtrans should not need to generate reference ‘.prf’ and spectral database files from scratch, only the ‘.pat’ files.

In summary:

Section 2 describes the atmospheric temperature, pressure and vmr ‘.prf’ profiles.

Section 3 describes the dust/aerosol ‘.prf’ profiles.

Section 4 describes the ‘.pat’ path files and the program Path.

Section 5 describes the detailed operation of the Radtrans program for the different spectral models available.

Section 6 describes the details of the post-processing programs Conv_spec, Li_spec and Pl_spec.

Finally Section 7 describes how line-by-line, band and correlated-k spectral databases are set up. Normally this will have already been done for you! In addition, Appendix A lists the Radtrans gas ID and ISO codes.

## 1.1 Location of Code and Reference Files

All source code is now under SVN management to maintain all versions of the software that might be released. The SVN central repository of Radtrans (and Nemesis) is http://scm.physics.ox.ac.uk/svn/radtrancode (please contact P. Irwin for details on how to access the files). The SVN repository contains the following subdirectories:

frecipes/ 	Numerical recipes Fortran programs. A makefile is included.

raddata/	Reference spectral data for use by Radtrans/Nemesis, including CIA tables, solar/stellar reference spectra, gas continuum spectra, lineshape factors, SVP curves and last, but definitely not least, a list of the current planets defined, together with their mass and radius, etc.

radtran/	Main Radtrans directory, containing a number of sub-directories. Several executables are defined, including Radtrans itself and a number of line data and correlated-k processing programs. The subdirecoties makefiles/ includes shell scipts for making the libraries in one go and for making all the Radtrans executables. The makefiles for each sub-directory are also included.

nemesis/	The Nemesis subroutines and programs. The Nemesis programs are built upon the Radtrans libraries in radtran/ so these must be compiled first.

idl/	Useful IDL programs for reading and plotting Radtrans/Nemesis calculations.

manuals/ 	The Radtrans and Nemesis manuals.

FOVgreg/	Library of routines for defining MCS Field of View. Only needs  compiling if you're going to run NemesisMCS or GenerateMCSspx.

All standard line data files are in the directory:

/home/oxpln98/plan/teanby/linedata

Useful planetary profile files (.prf files) are in the directory:

/home/jupiter/plan/plan2/oxpln5/radtran/profiles

Useful band data files (.ban files) are in the directory:

/home/jupiter/plan/plan2/oxpln5/radtran/banddata

Useful k-table files (.kta files) are in the directory:

/home/jupiter/plan/plan2/oxpln5/radtran/ktables

The code is currently using the Intel compiler on the departmental Linux machines and compiles on both 32 and 64 bit machines using the Intel Fortran Compiler. The code also compiles on Mac OSX, using gfortran.

# 2 Atmospheric T,P and VMR Profile Files (.prf)

Atmospheric profiles are read in from standard format ASCII ‘.prf’ files which contain the following information. The old format listed below consisted of 6 column blocks of data each with a descriptive header. Note that if more than three gases were included then these were listed in the next block and so on. This format has now been superseded by one where all the gas volume mixing ratios for a level are listed on the same line as height, pressure and temperature. Conversion of old format .prf files may be achieved using the program Convert_prf.

AMFORM

IPLANET  LATITUDE  NP  NVMR  MOLWT

ID(1) 	ISO(1)

:        	  :

:        	  :

ID(NVMR) 	ISO(NVMR)

"H        P           T           gas1         gas2         gas3"

H(1)     P(1)     T(1)     VMR(1,1)  VMR(1,2)  VMR(1,3)

:          :           :           :                 :                 :

:          :           :           :                 :                 :

H(NP)  P(NP)   T(NP)  VMR(NP,1) VMR(NP,2) VMR(NP,3)

"Additional gas profiles             4 to          ..."

VMR(1,4) VMR(1,5)    . .....VMR(1,NVMR)

:               :                              :

:               :                              :

VMR(NP,4) VMR(NP,5)  ...VMR(NP,NVMR)

AMFORM	Defines the format of the atmospheric profile. Above is the default format with AMFORM=0. If AMFORM=1, then it is assumed that at each level the VMRs add up to 1.0 and hence that the molecular weight can be calculated at each level. As a result of this for AMFORM=1, MOLWT is omitted from the list of parameters in the 2nd line.

IPLANET	Planet ID (3=Earth, 5=Jupiter etc)

LATITUDE	Planetocentric latitude (for calculating centripetal and non-spherical effects on gravity)

NP	Number of points in the profile.

NVMR	Number of gases whose volume mixing ratio (VMR) profiles are included in the file.

MOLWT	Mean molecular weight of the atmosphere in grams.

ID(1)	Radtrans identification code of gas 1. 
See Appendix 1 for definitions of Gas Codes.

ISO(1)	Radtrans identification code of gas 1's isotope. 0 = all isotopes in standard terrestrial relative abundance.

H(1)	Height in km of point 1 above reference level (ground for terrestrial planets, usually 1-bar level for gas giants)

P(1)	Pressure in atm at point 1

T(1)	Temperature in K at point 1

VMR(1,2)	Volume mixing ratio of gas 2 at vertical point 1.

Given the diverse sources for generating a planetary profile, it is usually necessary to generate new profiles off-line using bespoke software or an ASCII editor. However it is possible to initiate a new profile using Profile, a flexible manipulation program for the ‘.prf’ files which is now explained.

### 2.1 Profile

This is a general purpose manipulation program for ‘.prf’ files. At initiation, the program asks whether the user wishes to generate a new file or read in and adapt an existing file. If the user chooses to read in an existing file, the program will prompt for the filename and read in the data. If the user chooses to initiate a new file, the program will prompt for the appropriate data and generate the new profile. This first choice is really entirely up to you. For new profiles with only a few vertical points it is probably easier to enter the points manually using Profile. If there are many vertical points however, the user should probably familiarise himself with the ‘.prf’ format and generate a profile off-line. Should the user choose to enter the points manually however, the program will initially assume that you only know the temperature profile as a function of height. Profile will prompt for the number of vertical points, the planetary mass and radius and the mean molecular weight of the atmosphere. It will then prompt for the altitude and temperature of each point and generate the new profile. It will also compute a pressure profile assuming hydrostatic equilibrium.

Once a profile has been established, manipulation may begin. The program goes into a loop where at each stage the following menu is displayed:

---------------------------------------------------

A - display summary of profiles

B - add a VMR profile

C - remove a VMR profile

D - force VMR to saturation vapour pressure

E - compute temperature profile from hydrostatic equil.

F - compute pressure profile from hydrostatic equil.

K - compute height profile from hydrostatic equil.

H - output profiles and exit

I - multiply pressure by const (e.g. scale bar-atm)

P - output IDL compatable profile data

M - multiply vmr profile by constant

G - Map profile on to new pressure grid

W - Set water vmr profile to pr-um

V - output single gas VMR profile

Q - quit

---------------------------------------------------

command?

These options are now described:

A - display summary of profiles

This option simply displays a summary of the current profile to the screen. A typical output is:

profiles contain  51 points,  planetary nominal radius =  3400.000 km

5 volume mixing ratio profile(s) included

gas: 1  identifier: 2  isotope:   0

gas: 2  identifier: 1  isotope:   0

gas: 3  identifier: 7  isotope:   0

gas: 4  identifier: 5  isotope:   0

gas: 5  identifier: 3  isotope:   0

B - add a VMR profile

This option adds a gas volume mixing ratio to the profile. The new profile may be generated in one of three ways:

1) read in from an external file. The format of the .vmr file is in default ASCII and must contain the following information:

IDGAS		ISOGAS
VMRH(1)		VMR(1)
    :       		    :
VMRH(NV) 	VMR(NV)

IDGAS and ISOGAS are as before and identify which gas the file contains. The next lines then contain the volume mixing ratio at any number of heights. The heights do not have to correspond to those in the existing temperature/ pressure file and the file does not have to have the same number of points. Profile reads in the data and automatically detects the end of the file. The program then interpolates the points and maps them onto the heights of the stored temperature/pressure file

2) generate the new VMR profile assuming a constant mixing ratio with height which the program prompts for.

3) generate the new VMR profile in the same manner as 1) except reading the data directly from the keyboard and terminating with -1,-1.

C - remove a VMR profile

Simply removes a VMR profile from the stored profile.

D - force VMR to saturation vapour pressure

This option forces an existing VMR profile to be that of a saturated vapour pressure profile. The dependence of saturated vapour pressure (atm) on temperature (K) is assumed to be of the form:

The program prompts for the parameters a, b, c and d and then clips the VMR profile where required so that the partial pressure of the gas in question (PP = VMR(I,J)*P(I)) is always less than or equal to the saturated vapour pressure. A list of SVP parameters for commonly used gases may be found in …/raddata/SVP.dat

E - compute temperature profile from hydrostatic equil.

Given the height and pressure profile, together with the mean atmospheric molecular weight, the temperature profile is calculated from hydrostatic equilibrium.

F - compute pressure profile from hydrostatic equil.

Program prompts for the pressure level of zero height and then computes all pressures at higher altitudes according to hydrostatic equilibrium using the temperature profile and mean molecular weight of the atmosphere.

K - compute height profile from hydrostatic equil.

Program prompts for the height level of 1atm pressure and then computes all pressures at higher altitudes according to hydrostatic equilibrium using the temperature profile and mean molecular weight of the atmosphere.

H - output profiles and exit

Program prompts for a new profile filename and outputs modified data to a new .prf file.

I - multiply pressure by a const.

Self-explanatory! For example, pressures may be in bars and you want to convert them to atm.

P - output IDL compatable profile data

Outputs plotting data to an ASCII file which is readable by IDL routines such as plotprf.pro.

M - multiply vmr profile by constant

Multiply a specified VMR column by a user-defined constant.

G – Map profile on to new pressure grid

Self axplanatory.

W – Set water vmr profile to pr-um

Normalises water vapour VMR profile to the required column amount in units of precipitable microns.

V - output single gas VMR profile

Output new profile to disk containing only the P, T, H information and a single VMR profile.

# 3. Dust/Aerosol Profiles

Dust/Aerosol profiles are read in from external ‘.prf’ files which have a similar format to temperature/pressure ‘.prf’ files. The dust ‘.prf’ profiles are defined in terms of specific concentration, i.e. number of dust particles per gram of atmosphere. Up to ten different dust particle profiles can be defined at present in a single ‘.prf’ file. The dust profiles may be written by the program Dust_profile or by hand. At present Dust_profile can generate homogeneous, steady state and simplified cloud model profiles. These are defined as follows:

1. Homogeneous	= 	Constant specific concentration with height

2. Steady State		=	Developed for Mars, specific concentration is

constant at low altitudes and then falls off at a

rate depending on updraft/particle size etc.

3. Simple Clouds	=	Define base height, specific concentration at

base and scale height of vertically extended

cloud.

4. Better Clouds	=	Define base and cloud top pressures, fractional

scale height and required integrated optical depth of vertically extended cloud.

Other distributions can be hacked in as necessary or by manually editing an existing dust ‘.prf’ file. Note that to generate the dust profiles, Dust_profile must read the corresponding temperature/pressure profile. The format of a dust ‘.prf’ file is as follows:

NAME

NPRO,NDUST

DUST(1,1)  DUST(1,2)  . . .  DUST(1,10)

DUST(2,1)  DUST(2,2)  . . .  DUST(2,3)

"                "                . . .      "

"                "                . . .      "

DUST(NPRO,1)  DUST(NPRO,2)  . . .   DUST(NPRO,3)

Where:

NAME		CHAR		name of the file

NPRO		INT		number of vertical levels

NDUST	INT		number of dust types

DUST(I,J)	REAL		dust specific concentration of type J

at level I.

When Radtrans is run, it looks to see if any dust profiles have been read in and if so prompts the user for the name of the cross-section file. This file contains the extinction cross section of the particles in cm2 and the single scattering albedo for a number of wavelengths across the spectral interval. The value in each calculation bin is then determined by quadratic interpolation. The cross-section file may be written by either of the programs Write_xsec or Makephase or by hand. Makephase calculates the phase functions as well as the extinction x-sections of scattering particles. The spectral calculation programs then use the specific concentration profiles and cross-section spectra to calculate the optical thickness of each path due to dust for each spectral bin.

N.B. The default units of Radtrans are that the aerosol concentrations in the ‘.prf’ files are in units of specific density (number of particles per gram) which is a hangover from Mars radiative transfer days and is not very friendly. The default units of the ‘.xsc’ cross-sections are cm2. When Path is run, the number density of dust per layer is calculated and converted to units of particles/cm2. At the end of the day the units are somewhat arbitrary since Radtrans multiplies the number of particles/cm2 in each layer by the particle cross-section to calculate the optical path and it’s only the optical path that really matters! It is thus often (very) convenient to use the fourth model of Dust_profile to normalise the specific density profile of the cloud such that the values in the associated ‘.xsc’ file then equate to total optical depth of the cloud at different wavenumbers rather than the extinction x-section (cm2) of each particle .

# 4. Path

Once the atmospheric profiles of the required planet have been defined for a radiative transfer calculation, the atmosphere must be split up into layers, so that mean properties can be evaluated. A number of paths through these layers must then be defined. The program which performs this function is Path which writes a driver file (.drv) which defines all the details of the calculation for Radtrans and later Pl_spec. Path takes as its input a ‘.pat’ file (described in this section) which contains the details of the atmosphere, layers and paths and computes the absorber amounts, pressures, temperatures etc. in all the layers and then in all the paths. Path also sets the calculation types which are discussed in section 4.3. Radtrans calculates the transmission, absorption or emission spectra of all the paths defined in the ‘.drv’ file and outputs these to a binary ‘.out’ file. The calculation types are only used by the post processor Pl_spec to control how the path calculations are combined to calculate, for example, the radiance seen in the sideband of a PMR or, as another example the weighting function.

## 4.1 Layers

To perform radiative transfer calculations, the atmosphere in question must first be split into a number of vertical layers. Path reads the temperature/pressure/vmr profiles and the dust profile (if defined) and then using the subroutine layers.f splits the atmosphere into a number of discrete layers. It can do this in a number of ways:

1) Split by equal changes in pressure through atmosphere

2) Split by equal changes in log(pressure)

3) Split by equal changes in height

4) Split by equal changes in path length along viewing angle

The viewing angle  is defined as the angle of the line of sight of the calculation with respect to the normal at the bottom of the lowest layer.

The viewing angle is also used when calculating the mean properties of the layers. If the atmosphere is going to be used for a limb calculation, would be set at 90° and the layer properties calculated for a limb path to the bottom of the lowest layer. If the atmosphere is going to be used for a nadir calculation,  would be set at 0°. The path lengths through the layers are then determined in order that the mean properties of the layers may be calculated. This may be done in one of two ways:

1) The mean temperature, pressures (for each gas) and specific dust concentration q (particles/g) are set at the interpolated values at the middle of the layers as seen along the viewing angle. The total absorber amounts in the layer as seen along the viewing angle is then calculated as:

where P is the mean pressure (atm), T is the mean temperature (K) and (S1-S2) is the viewing angle path length (km) through the layer. The calculated absorber amount units are in (molecule/cm2) and hence the modified Boltzman constant is calculated as:

The dust amount in the layer along the viewing angle (particles cm-2) for each particle type is calculated as:

where q is the mean specific dust concentration (particles/g), U is the total atmospheric amount in the vertical layer (molecules/cm2), M is the mean molecular weight of the atmosphere in grams and N is Avogadro’s number.

2) The mean temperature and pressure in the layer are calculated as the absorber amount weighted means along the viewing angle path. The integration is performed using Simpson’s rule with the layer split into 101 points along the path. The absorber amount and dust amount are similarly integrated along the line of sight.

The paths through the layers are shown in Fig.4.1.



z0

z1

z2

s1

s0

Fig 4.1 Paths, Layers and Scaling Factors

Referring to that diagram, the first path s0 may be calculated from the cosine rule as:

Solving the quadratic in s0:

Other lengths are calculated in the same way. Once the mean pressures and temperatures of the layers, together with the gas absorber and dust absorber amounts have been calculated, the amounts are scaled back using the geometric scaling factor to simulate a vertical layer. For the example above, the scaling factor is:

Applying this scaling factor means that the layers can still be used for either nadir or limb calculations though of course it is better to calculate the layers in the manner closest to the majority of paths defined through the atmosphere.

At this stage cell ‘layers’ are also defined which simply contain the mean pressures, temperatures and absorber amounts of any instrument cells (e.g.for simulating PMR or SCR instruments). These ‘layers’ can be combined with atmospheric layers into paths.

## 4.2 Paths

The primary output from Radtrans are calculated transmission, absorption or emission spectra for a number of paths defined in the driver file.

For atmospheric paths, once the layer properties have been calculated, paths are determined by the subroutine atm.f. The viewing angle, bottom layer height and number of the lowest layer are defined in the ‘.pat’ file and atm.f selects which layers (atmospheric and cell) are to be used in the path (and in what order) and calculates the appropriate scaling factor using the previous equations.

Paths through cells may also be defined and combined as required with atmospheric paths to calculate PMR (Pressure Modulator Radiometer) or SCR (Selective Chopper Radiometer) outputs.

The number of paths in a driver file may vary from one for a simple transmission calculation to many tens of paths for weighting function calculations. However, for each path a model identifier is defined which determines the output of Radtrans for that path. The model identifiers (IMOD) are defined as:

0 = Transmission	(Atmosphere)

1 = Absorption (gives improved accuracy for low absorption)	(Atmosphere)

2 = Emission	(Atmosphere)

3 = Emission but uses source function at centre of current bin (much faster)	(Atmosphere)

4 = The total transmission computed during the last emission calculation (so that surface radiance may be added)	(Atmosphere)

5 = Single cell transmission	(Cell)

6 = PMR sideband transmission	(Cell)

7 = The last cell output path multiplied by the last ‘Atmosphere’ output path	(Combined)

N.B. Radtrans won’t check to see if last calculation was an absorption or transmission.

8 = The product of two previous outputs paths.	(Combined)

9 = The product of a single layer transmission and the output of the previous path (this is useful for speeding up calculation of transmission weighting functions)	(Atmosphere)

10 = As 2 but assumes transmissions calculated by single curtis-godson path	(Atmosphere)

11 = As 10 but uses source function at centre of bin	(Atmosphere)

12 = PMR Wideband transmission	(Cell)

13 = SCR Sideband transmission (1-Tc)	(Cell)

14 = SCR Wideband transmission (1+Tc)/2	(Cell)

15 = Scattering near-nadir calculation	(Atmosphere)

16 = Single-scattering calculation	(Atmosphere)

17 = Thermal emission into hemisphere	(Atmosphere)

18 = Thermal emission into hemisphere but using source function at centre of current bin	(Atmosphere)

19 = As 17 but using the Curtis-Godson approximation	(Atmosphere)

20 = As 18 but using the Curtis-Godson approximation	(Atmosphere)

21 = Net flux calculation (simple thermal emission)	(Atmosphere)

22 = Scattering limb calculation (calculates internal field)	(Atmosphere)

23 = Scattering limb calculation (precalc. internal field)	(Atmosphere)

24 = Net flux (up – down) calculation (scattering)	(Atmosphere)

25 = Upwards flux (internal) calculation (scattering)	(Atmosphere)

26 = Upwards flux (top) calculation (scattering)	(Atmosphere)

27 = Downwards flux (bottom) calculation (scattering)	(Atmosphere)

## 4.3 Calculation Types

The calculation types are defined by Path and are used only by Pl_spec to combine and plot the output of Radtrans. They are not used by the radiative transfer routines at all. Each ‘calculation’ is encoded in a single integer code ITYPE and passed via the driver file to Pl_spec. In general one bit in ITYPE corresponds to one of the logical variables where possible to simplify(!) evaluation of ITYPE. Calculation codes defined are:

•	Atmospheric Codes have bit 7 (128) zero

bit 6  (64) 1 if limb path

bit 5  (32) 1 if reflecting path (down then up)

bit 4  (16) 1 if transmission of single atmospheric path

bit 3   (8) 1 if Curtis Godson

bit 2   (4) 1 if weighting function

bit 1   (2) 1 if emission calculation

bit 0   (1) 1 if calculate 1 minus transmission

•	Non atmosphere codes have bit 7 (128) 1

128	Single cell transmissions

129	PMR - two pressure, two temperature approximation

130	PMR File. Pressures/temperatures defined at a

number of points in the cycle.

131	SCR

•	Combined cell/atmosphere codes

160 is a combined cell and atmosphere - produces combined output

161 is a reflecting path and cell - produces combined output

•	Reflecting layer atmosphere codes

200 is a reflecting layer calculation with no cells

201 is a reflecting layer calculation with cells

(Both these calculations allow selection of paths in Pl_spec.)

•	Scattering Calculation

256	Full doubling and adding scattering scheme

## 4.4 Path File Format

Path takes as its input simple ASCII path files (.pat) which contain a series of keywords and parameters. The ‘.pat’ files are block structured, each block starting with a keyword and ending with a blank line. These path files may either be generated directly using an ASCII editor or generated interactively using the program Write_path which prompts for the appropriate inputs.

An example of a typical ‘.pat’ file for Radtrans and an explanation of the possible keywords which may be processed by Path is given below. Note that the ‘interval’ and ‘line data’ formats have been slightly modified from [R10].

interval

1400 1495 5 5

1 1 5

spec data

~irwin/radtran/linedata/new/tim2

model ~irwin/pmirr_cal/tim2

dust model ~irwin/pmirr_cal/dust

dust spectra testdust

layer

nlay 35

layang 90.

layht 20.

laytyp 1

layint 1

atm

limb 1

therm

nowf

nocg

noabsorb

binbb

nobroad

reflatm

angles 45.0 30.0 8

nocg

noabsorb

cell

gases 1

1 0 1.0

length 10.

pmr twop

0.015449  292.0

0.020088  308.0

Defining the wavenumber interval (essential keyword)

The keyword is interval.

The first parameter line is the wavenumber minimum, maximum, output spacing and resolution in wavenumbers.

N.B. The final resolution parameter does not apply to LBL calculations (ICONV=0,1). For the adaptively integrated LBL model (ICONV=1) FWHM is hardwired to DELV as described in section 5.1.

The second parameter line is the spectral model type ICONV followed by two parameters PAR1 and PAR2 which define additional information for the particular spectral model used.

Defining the spectral data (essential keyword)

The keyword is spec data. The only parameter is a spectral data key file name (line data, band data, or k data) (‘.key’ extension, though this may be omitted).

The line data files are set up and manipulated by the programs: Makedb, Scan, Check_index, Li_lines, Pl_lines, Select, Cp_lines and Summary. These line data manipulation programs are described in section 7.

The band data files may be set up with the program Makeband.

The k data files may be set up with the program Calc_ntable.

Defining an atmospheric model (essential for atmospheric calculations)

The keyword is model and again the parameter is just the filename of the atmospheric model to use (‘.prf’ extension though again this may be omitted). In this case the filename is on the same line as the keyword.

The model files are produced and manipulated with the program Profile and have the extension ‘.prf’. They contain the pressure, temperature and gas VMR profiles as a function of height. The profile files are described in section 2.

Defining a dust or aerosol model (optional)

The keyword is dust model followed by the name of the dust/aerosol profile. The dust/aerosol profiles are explained in section 3.

Defining the dust optical properties (essential if dust is defined)

The keyword is dust spectra followed by the name of the dust/aerosol cross section (‘.xsc’) file. This file has the following form:

2

1900.00	1.0E-10	2.4E-10

0.5		0.1

2100.00	2.0E-10	3.0E-10

0.5		0.1

The first line contains the number of dust types. It must be equal to the dust types defined in the dust model.

Subsequent lines contain the cross-sections (cm2) for each particle type for as many wavenumbers as desired followed by the single scattering albedoes at the same wavenumber. The subroutine reading in the data automatically detects the end of the file.

Calculating atmospheric layers (essential for atmospheric paths)

The keyword is layer. Path splits the atmosphere into a number of layers. This is separate from calculations of the atmospheric paths so that you can have several paths through the same set of layers (e.g. for multiple tangent heights). There are multiple parameters on additional lines each defined by additional keywords. The keywords are:

nlay followed by the number of layers to split the atmosphere into.

layht followed by the height of the base of the lowest layer (km) relative to a reference level defined in the ‘.prf’ file.

layang followed by the angle from the zenith for splitting into layers. For example you'd use 0º for nadir calculations but 90º for limb. The stored layer properties are scaled by the cosine of this angle so that they approximate to the nadir case.

laytyp followed by an integer specifying the layering type.

0=split by equal changes in pressure over layers

1=split by equal changes in log pressure over layers

2=split by equal changes in height over layers

3=split by equal changes in path length at layang over

layers

layint followed by an integer specifying how to integrate absorber amount, pressure and temperature vertically over the layer.

0=use values at centre of layer

1=use curtis godson paths for a gas with constant mixing

ratio.

1 is usually better but 0 is useful for intercomparisons.

In the example above we have defined 35 layers starting from an altitude of 20km by splitting the atmosphere by equal changes in log(pressure). The mean pressure, temperature and absorber amounts of the layers are calculated using the Curtis-Godson approximation assuming that most of the paths will be limb-views to a tangent height of 20km

Calculating atmospheric layers with discrete cloud layers

The keyword is nlayer and the option is very similar to layer, but tailored for cloudy atmospheres such as that of Jupiter’s. Subsequent keywords are:

layint followed by an integer specifying how to integrate absorber amount, pressure and temperature vertically over the layer.

0=use values at centre of layer

1=use curtis godson paths for a gas with constant mixing

ratio.

1 is usually better but 0 is useful for intercomparisons.

layang followed by the angle from the zenith for splitting into layers. For example you'd use zero for nadir calculations but 90 for limb. The stored layers are scaled by the cosine of this angle so that they're always similar to the nadir case. You might use layang=0 for multiple limb paths or for simulating someone elses code.

nlaybot followed by the number of clear gas layers from the base of the profile to the bottom of the first cloud.

nlayg followed by the number of clear gas between cloud layers

nlaytop followed by the number of clear gas layers from the top of the last cloud layer to the top of the profile.

layht followed by the height of the base of the lowest layer (km) relative to a reference level defined in the .prf file.

ncloud followed by the number of main cloud decks

Subsequent line contains the cloud base height (km), cloud top height (km) and number of intermediate cloud layers.

Next lines contain the number of cloud particles per cm2 for a vertical path  for each sub layer.

Sequence then repeats for further cloud decks.

E.g. :

nlayer

layint   1

layang   0

nlaybot   8

nlayg   8

nlaytop   8

layht  -90.00

ncloud   3

-45.00000      -44.00000       1.000000

1.1200000E+13

-15.00000      -14.00000       1.000000

2.9099999E+10

16.00000       17.00000       1.000000

7.7999997E+10

If bi-modal clouds are required then instead of using nlayer, mlayer may be used which has an identical format with the exception that two columns now contain the number of cloud particles per cm2 for a vertical path for each particle type in each sub layer.

Defining an atmospheric path (essential for atmospheric calculations)

The keyword is atm. Additional parameters are defined on additional lines each defined by keyword. Most keywords can be negated by preceding them with a no. The keywords are

limb specifies a limb calculation and is followed by the number of the bottom layer to use. 1 is the bottom layer defined by layer above. Here paths are calculated to the tangent height and then beyond.

nadir specifies a nadir, or near-nadir calculation and is followed by the angle from the zenith and the bottom layer to use. For an instrument looking vertically downwards, the angle is set to 0°. For an instrument on the surface looking upwards the angle is set to 180°. In this case paths stop at the lowest layer. N.B. It is possible to add a third integer, IPZEN, on this line to indicate where the zenith angle is defined: 0) at bottom of deepest layer; 1) at the 0km altitude level; or 2) at the very top of the atmosphere.

(no)therm specifies a thermal emission calculation.

(no)hemisphere specifies a hemispherically-integrated thermal emission calculation using exponential integrals.

(no)wf specifies a weighting function calculation. Note that this forces notherm.

(no)cg specifies Curtis Godson paths are to be used.

(no)scatter specifies a full doubling and adding scattering calculation. If scattering is defined then when Radtrans is run there must exist a ‘.sca’ file containing details of the scattering calculation. This will be explained more fully later.

(no)netflux specifies if a net flux (Fup – Fdown) is to be calculated. This works with scattering on or off. Fluxes are computed at the base of each layer.

(no)upflux specifies if the upward flux only is to be calculated. This only works with scattering on. Fluxes are computed at the base of each layer.

(no)outflux specifies if the upward flux at the very top of the atmosphere is to be calculated. This only works with scattering on.

(no)botflux specifies if the downward flux at the bottom of the atmosphere is to be calculated. This only works with scattering on.

(no)absorb specifies that absorption rather than transmission is to be calculated for non thermal calculations. Needed for small absorptions when adaptive LBL integration (ICONV=1) is selected.

(no)binbb specifies that the Planck function is to be computed at bin centres (see interval above) not each wavenumber which is much faster but less accurate.

(no)broad specifies that an emission calculation is to be performed outside the RT calculation to simulate broad band calculations.

The defaults are limb, nowf, nocg, therm, binbb, noabsorb, noscatter, nonetflux, noupflux, nooutflux, nobotflux.

In the example atmosphere listed above we have defined the view path to be a limb path with a tangent height at the bottom of layer 1. We have specified a thermal radiance calculation where the Planck function is calculated for the centre of the bin only.

Defining a reflected atmospheric path

The keyword is reflatm. Additional parameters are defined on additional lines each defined by keyword. Most keywords can be negated by preceding them with a no. The keywords are:

angles followed by the incident and reflected angles and the number of the layer from which the reflection occurs (reflection happens at base of layer)

(no)cg Sets Curtis Godson flag as required

(no)absorb Sets absorption flag as required

Defining a gas cell

The keyword is cell. Additional keywords follow. These are:

gases followed by the number of gases in the cell followed on subsequent lines by the identifier, isotope and volume mixing ratio of each gas. The identifier is NOT always the same as HITRAN and is listed in Appendix A. Isotope zero includes all isotopes at terrestrial ratios. The RT routines correct line strengths for explicitly specified isotopes as if they were 100%

dop followed by a doppler shift for the cell (cm-1)

length followed by the cell length (cm)

sngl specifies single cells followed by the number of single cells.

Subsequent lines hold the pressure (atm) and temperature (K) (in that order) for each cell.

pmr twop specifies a Pressure Modulator Radiometer (PMR) calculation assuming a two pressure approximation

The next line holds the low pressure (atm) and associated temperature (K).

The next line holds the high pressure and associated temperature.

pmr file specifies that a PMR cycle is held in a file

The next line holds the file name and the file contains pressure and temperature for each point in the cycle on separate lines. The format of the files is free-form and Path automatically detects the end of the file.

scr specifies a Selective Chopping Radiometer (SCR) which alternately views the atmosphere directly or through a single cell. The next line holds the pressure and temperature of the cell.

N.B. Cell calculations can only be done on LBL calculations. Radtrans does not explicitly pick this up.

Defining single atmospheric paths

Instead of defining layers and atmospheres, single atmospheric paths may also be defined for various tests. Radtrans treats atmospheric and cell layers differently so this is not the same as defining single cell paths.

1.

The keyword is sngatm followed optionally by a file name. If there is no file name the input is read from the subsequent lines.

The first line should contain the keyword absorb or noabsorb as described earlier.

The following line contains the number of gases, pressure (atm), and temperature (K).

Each gas is defined on a subsequent line containing identifier, isotope, volume mixing ratio and amount (molecule cm-2). If the amount is < 1.e10 it is assumed to be a path length in km.

The next line contains the number of dust types NCONT. If NCONT > 0 then the next lines contain the name of the dust cross-section file and the dust amount in the path (number of particles/cm2).

Setting the error limit for adaptively integrated lbl calculations

The keyword is error followed by the limit (%) on the same line. This is the limit for terminating adaptive integrations. The default is 1%.

Defining how to process line data for lbl calculations

The keyword is process followed by the gas id, isotope id and an integer IPROC to define how to do the processing. At present two options are available:

IPROC=0	Default	Assume Voigt broadening for lines

IPROC=1			Assume Sub-Lorentzian line broadening (CO2)

IPROC=2			VanVleck-Weisskopf lineshape (far-IR)

IPROC=3			Rosenkrantz-Ben-Reuven lineshape for far-IR NH3

IPROC=4			Assume Lorentz-broadened lines

IPROC=5			Levy et al. (1994) PH3 line shape.

IPROC=6	Rosenkrantz-Ben-Reuven NH3 lineshape
same as IPROC=3 except default is voigt lineshape instead of sub-Lorentzian

IPROC=7	CO2 Sub-lorentzian line shape for Venus

IPROC=8	Burch CO2 Sub-lorentzian line shape

IPROC=81	Burch CO2 Sub-lorentzian line shape (renormalised)

IPROC=9	Hartmann Sub-lorentzian lineshape for methane modified for Titan by de Bergh et al.

IPROC=10	Hartmann et al. Sub-lorentzian lineshape for methane in a hydrogen atmosphere

Tidying the driver file

The keyword is (no)clrlay and tells Path to remove any unused layers before creating the driver file. clrlay is the default.

Combining atmospheric and cell paths

The keyword is (no)combine and tells Path to add additional paths combining all combinations of atmospheric and cell paths that have been defined. combine is the default. The combination is always at the end so you can't just combine those defined so far.

Fig.4.2 Generation of Radtrans driver files using Path

## 4.5 Running Path

Path is invoked by simply typing 'Path'. The program prompts for the name of the path (.pat) file and outputs the generated driver (.drv) file together with a name (.nam) file which is useful for running Radtrans in background. For example if the path file is called surf.pat, Path will generate surf.drv and surf.nam, the latter file is usually just one line long and containing the name of the driver file. However in some case it may also contain extra parameters not easily passed elsewhere such as whether an equilibrium ortho:para H2 fraction is assumed or a deep 3:1 ratio [R15].

## 4.6 Changes in driver file format from Genlbl

Radtrans has been designed to look as much as possible like the previous line-by-line model Genlbl in order that old calculations can be repeated using the same driver files. There are only three ways in which the driver files are different:

1) Since the treatment of dust/aerosols has been changed, NCONT must be set to 0 if there is no dust instead of 1 as in Genlbl.

2) If dust profiles are defined then an extra line follows the line containing ICONV and NCONT which contains the name of the dust extinction cross-section file.

3) Dust amounts in the driver file are now in units of particles/cm2 instead of optical depth.

The lines which contain details of any required filter profiles are still present and Radtrans will use them in the same way that Genlbl does if required. However it has only been left in to reduce the number of changes between the two systems and it is now recommended that this function should be performed as part of the post-processing in Conv_spec.

## 4.7 Non-LTE

User specified NLTE parameters are also included in Radtrans. To use the facility the driver file must again be manually edited. Either before, after or instead of the hydrogen absorption parameters insert the line:

NLTE 0

The number specified is a code to indicate the type of NLTE required: At present there are two possibilities:

0	Perfect LTE

1	Assume NLTE is as that estimated by Glenn Orton for

Saturn and Titan. Scaling for Planck function is:

# 5. Radtrans

As outlined earlier in Section 1, Radtrans calculates transmission, absorption or emission spectra for the paths defined in a ‘.drv’ driver file constructed from a ‘.pat’ path definition file by the program Path. Spectra are calculated in wavenumber space only and the calculated spectra are written to binary ‘.out’ files. Hence the output files must be processed using one of the three post-processors Conv_spec, Li_spec and Pl_spec which are described later and which may transform the spectra to wavelength space (if required) and write the outputs in ASCII format. The spectral details of the calculation are given in the first line of the ‘.drv’ driver file where the wavenumber minimum, spacing, number of points and bin width are defined. The subsequent line contains two extra parameters PAR1 and PAR2 whose meaning depends on the spectral model required which is defined by the integer ICONV. Table 5.1 summarises the model options which are dealt with more fully in the following sections.

| Spectral Model | ICONV | PAR1 | PAR2 |
| --- | --- | --- | --- |
| Line-by-Line (LBL) Calculations |  |  |  |
| Line by line model - monochromatic | 0 | WING | VREL |
| Line by line model - integrated | 1 | WING | VREL |
| Band Model Calculations |  |  |  |
| Non-overlapping line model (linedata) | 10 | EquivW code | - |
| General random band model (linedata) | 11 | EW | 1 |
| Malkmus-Lorentz band (linedata) | 11 | EW | 2 |
| Goody-Lorentz band (linedata) | 11 | EW | 3 |
| Godson-Lorentz band (linedata) | 11 | EW | 4 |
| Malkmus-Lorentz band (band data) | 12 | - | - |
| Goody-Lorentz band (band data) | 13 | - | - |
| Godson-Lorentz band (band data) | 14 | - | - |
| Goody-Voigt band (band data) | 15 | - | - |
| Correlated-k Calculations |  |  |  |
| Correlated-k model (linedata) | 20 | WING | VREL |
| Malkmus-Lorentz Corrk (band data) | 21 | 1,2,3 | - |
| Goody-Voigt Corrk (band data) | 22 | 0,1 |  |
| Exponential Sum model (not recommended) | 23 | 0,1 | - |
| Correlated-k model (look-up table) | 24 | - | - |

Table 5.1 Radtrans Spectral Models

Heavily shaded options not yet completed

## 5.1 Line by Line Models	ICONV<10

Here:

PAR1=WING

PAR2=VREL

The spectral region defined in the path and driver files is extended to VMIN-VREL to VMAX+VREL, to cover line wings from lines outside the interval and divided into bins WING wavenumbers wide (Fig 5.2). Unlike Genln2, Radtrans ignores all lines outside the range VMIN-VREL to VMAX+VREL. Line data is stored in blocks corresponding to the WING-sized bins. At a particular wavenumber, the absorption by lines from the bin at that wavenumber and the two adjacent bins are treated explicitly. Absorption by lines from other bins is calculated by calculating the contribution to the middle and each end of the bin and fitting a quadratic since at this distance from the line centres it is assumed that we are in the Lorentzian (and thus quadratic) tail of the line shape (Figure 5.1). This greatly increases the calculation speed.

Fig 5.1 Treatment of lines by Radtrans line-by-line code.

Line-by-line calculations may either be performed monochromatically (ICONV=0) or integrated over square bins of width DELV using a cunning adaptively integrating Simpson’s rule scheme (ICONV=1). For the example path file given, Radtrans will integrate the spectrum over 5 cm-1 (i.e. DELV) wide regions centred on the required output wavenumber. The first region is thus from 1397.5 to 1402.5 cm-1 giving the average transmission or radiance at 1400 cm-1. [Note - the integration steps and the bin sizes are not related. Hence an integration may cover many bins or just one.]

The adaptive integration scheme works by first performing a Simpson’s rule integration over the square bin of width DELV using 20 points. The number of intervals is then doubled and the integration evaluated again. The scheme tests for convergence by comparing the last two estimates. If they agree to within the convergence limit (set by error in the ‘.pat’ file and defaulting to 1%), the integration stops. Otherwise the number of integration steps is doubled and so on.

### 5.1.2 Line handling in Radtrans

Previous releases of Radtrans included a program Radtran which used a clever line-binning scheme for LBL calculations to reduce the amount of data held in memory. However this scheme considerably increased the complexity of the code, making debugging and error analysis difficult. In addition, once the internal memory limit was exceeded a bin-swapping procedure was adopted which greatly increased the computation time. Hence it was decided to no longer support Radtran. Instead Radtrans has been developed which behaves exactly in the same way as Radtran, except that all the lines for all the bins are read in first. If the required calculation covers more than 10000 lines, which is currently the limit of Radtrans, the calculation range must be split up into separate spectral ranges.

## 5.2 Band Models	9 < ICONV < 20

Band models are useful for low- to medium-resolution spectral modelling. They represent a significant simplification over line-by-line models in that relative position of molecular absorption lines in a spectral interval is ignored. In the models included here it is assumed that the lines are randomly distributed across the interval and thus the equivalent widths of the lines (i.e. integrated absorptions) may simply be added. The models are generally not so accurate as line-by-line models but have the advantage that they are much faster and in some cases, where line data is not well known, are the only option available. While dust absorption is easily included, dust scattering is not possible with band models (since they must use the Curtis-Godson approximation), which is why the correlated-k models have also been developed. For a full discussion of band and correlated-k models, see [R1]. Two generic types of band models are included, those that use line data and those that use band data.

### 5.2.1 Band Models using Line data (ICONV=10, ICONV=11)

For every central output wavenumber, each line in the integration range of width FWHM is read in and the equivalent width calculated before the mean transmission is calculated using a number of possible approximations.

The equivalent width W of each line may be assumed to be that of:

a) 	a Lorentz line only WL.

b)	a Lorentz line combined with a Doppler line using the equation  to simulate Voigt lines. (Here S is the line strength and m is the absorber amount)

c)	interpolated from a Voigt width look-up table [R2].

Both Lorentz and Doppler equivalent widths may be calculated using either the Weak or Strong assumptions or a combined width [R2].

Once the line equivalent widths have been calculated, the path transmissions may be calculated using

a) the non-overlapping line approximation where

b) the general random band model where

c) the Malkmus Lorentz band model [R3]. Here

where

is the weak limit of the optical thickness and  is the strong limit.

d) the Goody Lorentz band model [R3] where

e) the Godson Lorentz band model [R3] where:

where

and

and I0 and I1 are Bessel functions.

For these models, the path file parameter PAR1 contains the code for calculating the equivalent width WIDMOD. WIDMOD is an integer whose three digits when converted to base 3 are the codes COM_MOD, LOR_MOD and DOP_MOD respectively. For example, the number 7 is 021 in base 3 which sets COM_MOD=0, LOR_MOD=2 and DOP_MOD=1. These codes are identified below.

COM_MOD. This code defines whether a voigt line shape is to be assumed and if so how:

COM_MOD = 0, Equivalent width used is that of a Lorentz line only

COM_MOD = 1, Equivalent width used is that of a Lorentz line combined with a Doppler line using the equation  to simulate Voigt lines

COM_MOD=2, Equivalent width used is interpolated from a voigt width look-up table [R2].

LOR_MOD. This code defines how the Lorentz equivalent width is to be calculated.

LOR_MOD=0, Equivalent Lorentz width is assumed to be Weak.

LOR_MOD=1, Equivalent Lorentz width is assumed to be Strong

LOR_MOD=2, Equivalent Lorentz width is assumed to be intermediate between weak and strong as calculated by [R2]

DOP_MOD. This code defines how the Doppler equivalent width is to be calculated.

DOP_MOD=0, Equivalent Doppler width is assumed to be Weak.

DOP_MOD=1, Equivalent Doppler width is assumed to be Strong

DOP_MOD=2, Equivalent Doppler width is assumed to be intermediate between weak and strong as calculated by [R2]

The band models are defined as follows:

Model	ICONV	PAR1	PAR2

Non-overlapping Lines	10	EW code	Undefined

General random band	11	EW code	1

Malkmus-Lorentz	11	EW code	2

Goody-Lorentz	11	EW code	3

Godson-Lorentz	11	EW code	4

### 5.2.2 Band Models using Band data 	(11<ICONV<20)

These models used band data generated off-line either by the program Makeband or from external sources. Radtrans uses these parameters to calculate the spectral transmission or radiance using one of four models [R4] (note that the absorber amount (in units of molecule-cm-2) must first be multiplied by the factor 10-20 to cancel the factor in ). The available models are:

Model	ICONV

Malkmus-Lorentz	12

Goody-Lorentz	13

Godson-Lorentz	14	Not completed

Goody-Voigt	15

The additional parameters PAR1 and PAR2 are not used for these models.

ICONV=12 	Malkmus-Lorentz

The Malkmus-Lorentz transmission of a single gas is given by:

where:

, where m is the absorber amount and

The Lorentz width is found from the equation where q is the volume mixing ratio of the active gas and the temperature dependence exponent is assumed to be 1/2.

The mean absorption for the conditions of the gas is given by  where qr is the rotational partition function which is 1.0 for linear molecules and 1.5 for non-linear polyatomic molecules [R5]

If more than one gas contributes to the absorption, the lines are assumed to overlap randomly and thus the equivalent widths are simply added.

Expressing  , where W is the equivalent width, then in the weak limit:

as  ,

If we assume randomly oriented lines, then in the weak limit:

In the strong Lorentz limit:

as  ,

Again assuming randomly oriented Lorentz broadened lines:

Thus equating the weak limits:

and equating the strong limits:

ICONV=13 	Goody-Lorentz

The Goody-Lorentz transmission is given by:

where A and B , the temperature dependencies of  and the mean absorption are all calculated in the same way as for the Malkmus-Lorentz model. However, here the statistical definition of  is different.

Equating the weak limits we again obtain:

but in the strong Lorentz limit:

as  ,

which is equivalent to  for randomly distributed Lorentz lines. Thus equating the limits:

Which is a factor of 4 bigger than for the Malkmus-Lorentz model.

ICONV=14 	Godson-Lorentz

Still not completed! (and probably never will be!)

ICONV=15 	Goody-Voigt

The Goody-Voigt transmission is given by the integral:

where

This function has to be integrated numerically.

### 5.2.3 Summary of Radtrans Band Models

The band models available in Radtrans are:

Spectral Model	ICONV

Non-overlapping Lines using line data	10

Random Band Models using line data	11

Malkmus-Lorentz band model (band data)	12

Goody-Lorentz band model (band data)	13

Godson-Lorentz band model (band data)	14 (not complete)

Goody-Voigt band model (band data)	15

## 5.3 Correlated-K models 	19 < ICONV < 30

Correlated-k models share the same speed advantages of band models but may also be used for scattering calculations ([R1], [R6] and [R7]).

As far as band transmission is concerned [R1], it is of no importance where in a frequency interval , a particular absorption coefficient occurs; it is sufficient to know what fraction of the frequency domain f(k)dk, is occupied by absorption coefficients between k and k+dk. It is thus possible to write the mean transmission function (where the transmission is a function of the absorber amount) for a homogeneous path in either of the two forms:

or

The form of this last equation is a standard Laplace transform and thus we can define the mean transmission function of a path to be the Laplace transform of the absorption coefficient frequency distribution:

Similarly, the absorption coefficient frequency distribution may be defined as the inverse Laplace transform of the transmission function:

Looking at the form of the equations it becomes apparent that the change of variable and integration space need not be restricted to evaluation of the transmission. In general if G is any analytic function where:

then the following transformation applies:

Herein lies the principle advantage of k-distribution models, once the absorption coefficient frequency distribution of the gas absorption is determined, calculations such as multiple scattering may also be performed in k-space, and as we shall see, this space is much more amenable to efficient and fast calculations.

We now define the cumulative frequency distribution g(k):

Since f(k)dk represent fractions of the interval where the absorption coefficient lies between k and k+dk, the limit of this integral as k tends to infinity is unity. If we invert this function to express k as a function of g:

Then the mean transmission may then be written:

The cumulative frequency distribution g(k) is a smooth monotonic function and we define its inverse to be the k-distribution which also varies monotonically and is thus easily integrated with comparatively rough quadrature. Dividing the space between 
g = 0,1 into N intervals such that the variation of k with g is sufficiently well sampled, the mean transmission of a path may be calculated from:

where  is the mean absorption coefficient in the ith g-interval.

For band models, radiance calculations may only be performed using Curtis-Godson equivalent path approximation, thus precluding scattering. However when k-distribution models the Curtis-Godson approximation is not the only one that may be used. Referring to [R6], for an atmosphere split vertically into a number of layers, for a particular spectral interval there is a very good spectral correlation between the high and low absorption features of different layers. Hence, to a good approximation, the combined transmission of M layers may be calculated as:

This is the correlated-k approximation and the degree of correlation is in most cases exceptionally good, and usually better than the Curtis-Godson approximation. It should be noted that to use the correlated-k approach, the number and width of g-intervals must be the same for all layers.

### 5.3.1 Correlated-k model using line data 	(ICONV=20)

This option of Radtrans uses line-by-line routines to calculate the absorption coefficient spectrum of each layer and then numerically extracts a high-resolution k-distribution function which is then re-sampled to the required number and widths of the g-intervals.. This process is of a similar speed to a normal line-by-line transmission calculation but has the advantages of accurate determination of g(k) and the amenability of using the k-distributions for fast multiple scattering calculations. [To numerically calculated g(k), the absorption coefficient spectrum is basically analysed to evaluate what fraction of the spectral interval has absorption coefficients in the range k to k+dk.]

### 5.3.2 Malkmus-Lorentz Band Correlated-K 	(ICONV=21)

The Malkmus-Lorentz model is particularly suitable for application of k-distribution methods since the k-distribution may be determined analytically from the band model parameters. Expressing the Malkmus-Lorentz band model in the form:

(where  and ) then the absorption coefficient frequency absorption is given by:

with a peak value at:

The cumulative frequency distribution may also be expressed analytically as:

where ,  and where erf(z) is the error function:

Unfortunately, no analytical solution for the k-distribution which is the inverse of g(k) exists and so this must be found by iteration.

As mentioned earlier, once g-intervals have been defined, the mean absorption coefficient is most easily determined as the arithmetic mean. However when this is done the resultant mean transmission is not exactly that of the equivalent band model but depends on the number and width of the g-intervals. However the mean transmission can be forced to be exactly the same as the equivalent band model if we use the cumulative transmission function h(k) which is dependent on the particular path being calculated:

For the Malkmus-Lorentz distribution, this function exists in a closed form:

where ,  and where  is Malkmus-Lorentz transmission function. Once the limits of k are found for each interval of g, the mean absorption coefficients are defined as transmission weighted averages:

or

Since the mean absorption coefficients defined here are weighted averages for a specified absorber amount, the resulting histogram of absorption coefficients is necessarily absorber dependent. Because of this, the transmission computed by

remains equal to the Malkmus-Lorentz transmission regardless of the number of g-intervals used. This removes any errors in the calculation due to fitting k-distributions to band models. However the number and width of the g-intervals obviously affects the accuracy of any multiple scattering calculations. In practice it is best to use g-intervals which are unequal in width but which provide approximately equal ratios in absorption coefficient strength. In the case of multiple scattering and vertical inhomogeneity then for a given number of intervals, optimum accuracy is obtained when the g-intervals are more closely spaced near g = 0 and the strong absorption end near g = 1.

The additional parameter PAR1 here defines further details of how the Malkmus-Lorentz correlated-k model is to be calculated:

PAR1=1 	a) The mean absorption of each sub-interval is the simple arithmetic mean of the absorption coefficient in the interval.

b) Overlapping gases in the intervals are combined using the weak and strong limits.

This second part may be explained as follows. To determine the B and S Malkmus-Lorentz parameters of the interval when there is more than one active gas, the B and S parameters for each gas are combined in a manner which is consistent with the band model assumption of uncorrelated randomly overlapping absorption lines. Consider, for example, two overlapping gases that are characterised by band model parameters Ba, Sa and absorber amount ua, and Bb, Sb, and ub respectively. The statistical parameter definitions of these parameters in the weak and strong limits imply:

and

PAR1=2 	a) The mean absorption of each sub-interval is the absorber amount weighted mean using the cumulative transmission function.

b) Overlapping gases in the intervals are combined using the weak and strong limits.

PAR1=3 	a) The mean absorption of each sub-interval is the absorber amount weighted mean using the cumulative transmission function.

b) Overlapping gases in the intervals are combined by summing the optical depths.

Combining the band parameters statistically is reasonable if the absorption of the overlapping gases is comparable with similar degree of saturation. However in the extreme situation where one is strongly absorbing and the other is weakly absorbing, the square root dependence of the product SB can lead to greatly overestimated values of B. In these situations it is better to add the effective Malkmus-Lorentz optical depths of the two gases calculated with:

and then use the equation:

to determine B. The S parameters, which add linearly, are not adversely affected by the absorber mismatch.

In all three cases, the integral over g is performed by the simple sum solution.

### 5.3.3 Goody-Voigt Band Correlated-K	(ICONV=22)

For calculations in the Jovian atmosphere, the spectral properties of methane are important. Unfortunately the line data for CH4 is not very reliable in the near-IR (although is improving steadily) and so band models are often used where the band parameters are fitted to laboratory measurements of methane spectra. [R4] found that the best fit to the laboratory measurements was the Goody-Voigt band and to use this data in a scattering model of Jupiter's atmosphere requires a Goody-Voigt correlated-k model. Unfortunately, k(g) cannot be determined analytically from the Goody-Voigt parameters but must be determined from a numerical Laplace transform of the transmission spectrum. This may be done in one of two ways:

PAR1 = 0, 	k(g) is the mean absorption coefficient over the 			interval

PAR1 = 1,	k(g) is weighted by the cumulative transmission

function to get close agreement between correlated-k

and band calculations

### 5.3.4 Exponential Sum Model	(ICONV=23)

Not been updated fro some time – use with caution! Performs least squares fit to the transmission functions calculated with band models to derive the exponential sum k-coefficients directly. This is usually better done off-line with the program, Band_ktable and the k-tables then used with the ICONV=24 model below. There are two models depending on PAR1:

PAR1 = 0,	k(g) calculated from Malkmus-Lorentz transmission curve

PAR1 = 1,	k(g) calculated from Goody-Voigt transmission curve

Model only still exists for historical completeness. Recommend using ICONV=24 instead.

### 5.3.5 k-Distribution Look-up Table	(ICONV=24)

Identical to ICONV=20, except the k-coefficients are calculated for each gas BEFOREHAND over a representative grid of pressures and temperatures using either Calc_ntable for line data, or Band_ktable for band data. Overlapping gases are combined as per [R8]. Look-up tables calculated with Calc_ntable and Band_ktable may be inspected using the program Read_table.

## 5.4 Running Radtrans

To run the program Radtrans, the atmospheric information in the ‘.prf’ profile files and spectral details must be combined into a driver file (.drv) using the program Path as previously described in Section 4. In addition, the user must supply the ‘.cia’ and ‘.fla’ files to define various model options. These files are described in the Nemesis manual. If the driver file has the name ‘surf.drv’ then Radtrans is invoked (in background) using the line:

Radtrans < surf.nam > surf.prc &

The binary output spectra are sent to the file surf.out while any processing information is sent to surf.prc. In addition a log file is written, called in this case surf.log, which contains a full listing of the run details for reference. An overview of the processing software was shown earlier in Fig. 1.1:

If one of the correlated-k models has been selected then there must also exist a ‘.gin’ file which contains the desired g-ordinates and weights for a Gauss-Legendre integration. This file has the same name as the driver. A typical ‘.gin’ file has the following format:

10

1.3046736E-02	3.3335671E-02

6.7468315E-02	7.4725673E-02

0.1602952      	0.1095432

0.2833023     	0.1346334

0.4255628       	0.1477621

0.5744371      	0.1477621

0.7166977      	0.1346334

0.8397048      	0.1095432

0.9325317      	7.4725673E-02

0.9869533		3.3335671E-02

where the first integer is NG, the number of g-ordinates and the subsequent columns contain the g-ordinates and weights respectively.

Finally if a scattering path is defined, then a ‘.sca’ file must also be present whose format is described in Section 8.5.2.

# 6. Post Processing of Radtrans Output Spectra

Three main programs are available for processing the binary output data of Radtrans:

Conv_spec

Li_spec

Pl_spec

In addition, the programs Aground and Diskav may also be used to produce more specific output (described below).

### 6.1 Conv_spec

The program Conv_spec convolves the output spectra with user specified instrument functions . The program is very similar to Conv described in [R10] but slightly more general and manipulable.

The output from Radtrans are binary spectra with interval DELV cm-1 where the calculated quantities are either averages over a square bin of width FWHM centred the output wavenumber, or for ICONV=0, pure monochromatic spectra. For example, if ICONV  0, if VMIN = 2000 cm-1, DELV = 5 cm-1 and FWHM = 5 cm-1 then the first output at 2000 cm-1 will be the average from 1997.5 to 2002.5cm-1. Conv_spec, takes these spectra and applies one of the following functions:

1	Square bin convolution

2	Triangular bin convolution

3	Multiplication by a Filter profile followed by square

bin convolution

4	Sinc function convolution

The convolutions shapes are of user-specified widths and may be applied in either wavelength or wavenumber space. Conv_spec limits the convolution to wavenumbers where the convolution shape is within the originally defined interval so output ‘.ave’ spectra always contain less points than the original .out spectra. The centres of these convolution shapes is at the same wavenumbers as the output spectra and Conv_spec determines the average of the convolution shape over each bin and calculates the weighted mean. If the convolution shape covers half a bin, then an appropriate fraction of the bin is added. This of course assumes that the absorption is evenly spread over the bin which may or may not be a good idea. In general it is better not to use fractions of bins so if you want to calculate an output spectra with triangular convolution of FWHM=10cm-1, for example, set DELV = 6.666 and FWHM = 6.666 cm-1 .

A typical input sequence is shown below:

driving file name? :testc1

enter FWHM, wavespace [0=wavenumber, 1=microns]:10,0

Enter ICONV [1=square, 2=triangle, 3=filter, 4=sinc ] :2

### 6.2 Li_spec

The program Li_spec reads in either a direct output or convolved spectrum and writes the data to the screen or an ASCII file. The user is guided interactively through its operation.

### 6.3 Pl_spec

The program Pl_spec again reads in either a direct output or convolved spectrum and outpots spectra to a an ASCII file suitable for display by the IDL routine plot_radtran.pro. After the spectrum has been read in, the program gives the user the following options:

A - display calculations

B - display paths

C - display layers

D - plot a spectrum for a path

E - plot results of a calculation

Q - quit

To illustrate the effects of the above options, consider the spectrum generated using the following driver file:

original name of this file: testc1.drv

1900.000   5.00000        21   5.000     :Vmin dV Npoints FWHM (BAND)

0.000   2.000       : Additional codes PAR1 and PAR2

5jupiter5

12     0             : type of convolution/model & NCONT

1   1   1           : number of layers, paths and gases

6                                    : identifier for gas 1

1   0                              : isotope ID and process parameter

format of layer data

layer baseH  delH   baseP      baseT   totam       pressure    temp   doppler

absorber amounts and partial pressures

continuum points if any

1   0.000   0.000 0.00000E+00   0.000 0.36717E+25 0.10000E+01 296.000  0.0000

0.36717E+25 0.10000E+01

1   0 0.10000E-01            : Nlayers, model & error limit, path  1

1    1   0.000  0.10000E+01  :     layer or path, emision temp, scale

1                                    : number of filter profile points

0.00000E+00     0.000                  : filter profile point   1

testc1.out

1                                     :number of calculations

128   2   0   0                        :type and # of parameters for calc  1

1

1

A - display calculations

This option simply outputs to the screen the number of different calculations being done and the number and value of the parameters defining it. The output in this case is:

there are  1 calculations

--------------------------------------------------

Calculation :  1

Cell calculation

Single cell calculations

Number of cells =   1

Path:   1 Layer:   1

--------------------------------------------------

From the driver file, the first calculation has ITYP=128 which corresponds to a single cell transmission.

B - display paths

This option summarises the paths for which spectra have been calculated. The output in this case is:

path: 1  model type: 0,   1layers included

1

---------------------------------------------------

Thus this driver contains just one path which is a cell path for which the transmission is calculated (IMOD = 0) and comprises just one layer, LAYER=1.

C - display layers

This options summarises the layers used in the spectral calculations and the number of gases used. The output in this case is:

there are  1 gases

gas: 1  identifier:  6   isotope id:    1

--------------------------------------------------

layer: 1  pressure: 0.10000E+01bars  temp: 296.00K

gas amounts

0.36717E+25

--------------------------------------------------

D - plot a spectrum for a path

This option plots the calculated spectra for any of the paths. The option has an interactive input to define which path spectrum to output and in what form. A sample session for the ‘.drv’ file above is shown below.

absorbtion plot? [1-output] :y

Plot against wavenumber (1) or microns (2) :1

log plot? [Y/N] :n

1945.000       1900.000

wavenumber ( x ) limits are:   1900.000  2000.000

y limits are:  0.00000E+00 0.88040E+00

force plotting axes? [y/n] :n

Different options appear for different spectral calculations, e.g. brightness temperatures may be plotted for radiance calculations.

E - plot results of a calculation

This option plots the results of calculations defined by the ITYPE integers. In most cases, the calculation output is simply one of the calculated path spectra. However, this option also plots the PMR transmissions/emissions and weighting functions. This is the option which should be most commonly used. An example input session is shown below.

there are   1 calculations

plot which one? :1

------------------------------------------

Cell calculation

Single cell calculations

Number of cells =   1

Path:   1 Layer:   1

------------------------------------------

Cell calculation

Single cell transmission

There are   1 paths :

1

Enter desired path :1

absorbtion plot? [1-output] :y

Plot against wavenumber (1) or microns (2) :1

log plot? [Y/N] :n

1945.000       1900.000

wavenumber ( x ) limits are:   1900.000  2000.000

y limits are:  0.00000E+00 0.88040E+00

force plotting axes? [y/n] :n

As can be seen, the option first prints the details of the calculation in the same way as for option A.

A typical input sequence if the calculation is for reflecting layers is:

there are   5 calculations

plot which one? :5

------------------------------------------

Reflecting Layer Calculation

Pure atmospheric paths

Number of atmosphere calculations =   1

Number of reflected atmosphere calcs =   3

Atmospheric calculations are:

4

Reflection Atmosphere calculations are:

1

2

3

------------------------------------------

Cell calculation

Reflecting layers calculation

Pure atmosphere paths

Number of atmosphere calculations =   1

Number of reflected atmosphere calcs =   3

Atmospheric calculations are:

4

Reflection Atmosphere calculations are:

1

2

3

Enter required number of reflecting calcs :3

Enter reflecting atmosphere calc :1

Enter albedo of layer :0.9

Enter reflecting atmosphere calc :2

Enter albedo of layer :0.5

Enter reflecting atmosphere calc :3

Enter albedo of layer :0.1

Enter required number of atmospheric calcs :1

Enter atmosphere calc :4

Enter distance from sun (AU) :5

Enter surface temperature of Sun (K) :6000

plot brightness temperature? :y

Plot against wavenumber (1) or microns (2) :1

log plot? [Y/N] :n

1998.850       1700.000

wavenumber ( x ) limits are:   1700.000  2299.850

y limits are:  0.00000E+00 0.26876E+03

force plotting axes? [y/n] :n

The reflecting atmosphere calculations correspond to reflatm in the path file and the atmosphere calculations correspond to atm.

The calculated radiance spectra in this case are determined as:

where:

Ra is the calculated thermal radiance from the atmosphere alone.

ti is the transmission from the sun to the ith reflecting layer and back to the instrument.

A is the albedo of the reflection layer

I0 is the incident solar flux:

where 0 is the angular radius of the sun at 1 A.U. and D is the distance from the sun in A.U.

### 6.4 Aground

Adds ground emission spectrum multiplied by an atmospheric transmission spectrum to an atmospheric emission spectrum. Assumes .out file contains both the atmospheric thermal emission and the transmission spectrum to the ground and combines them appropriately given the surface temperature. This program basically does the same job as the old Addground program without the use of intermediate ASCII files.

### 6.5 Diskav

Program for numerically simulating disk-averaged spectra. The Radtrans ‘.out’ file is assumed to contain thermal emission spectra calculated at a number of emission angles from a .pat file written by Write_diskpat. A Guassian Quadrature over emission angle is assumed with the angles and weights defined by getquad.f. Diskav simply combines the output spectra (which may also include ground radiances) using the weights specified by getquad.f to generate disk-averaged spectra. This program has been somewhat superseded by the introduction of the analytical exponential-integral method of achieving the same result.

# 7 Setting up Spectral Data

## 7.1 Line data

A number of programs are available for setting up and manipulating direct access line databases. An overview of the software is shown in Fig.7.1.

Fig. 7.1. Overview of Linedata Manipulation Software

### 7.1.1 Makedb

This is the central program of the system and transcribes sequential-access line databases (eg HITRAN or GEISA) from files as copied from tape into direct access files containing one transition per record. The records are in FORTRAN character format as in the original database and conversion to ASCII from EBCDIC is allowed. An index file is also produced consisting of one record corresponding to each record (transition) in the database. This record contains a pointer to the database record holding the next transition for this species (of any isotope). The program also produces a key file summarising the following information:

the name of the database file

the name of the index file

the name of the file describing the gas_info file, which lists the gases known to the Radtrans software and also includes a cubic polynomial description of the partition function for each isotope (see section 7.1.13), together with the relative abundances of these isotopes assumed in line database. The line strengths of the different isotopes are already multiplied by these relative abundances in the line database and so if you want to see the line strengths of particular isotopes, the listed strengths need to be divided by the relative abundance.

the name of the file containing the translation between isotope identifiers in the database used and those used by Radtran

Thus only this key file has to be specified to identify the database. Since HITRAN and GEISA use slightly different gas identifiers, Makedb and all the subsequent Radtrans software uses its own identification system, based on HITRAN so that both data formats can be transcribed. The gas identification system used is shown in Appendix A. The local gas identifier agrees with HITRAN IDs as far as possible (Jan 1988) with extra IDs for gases not in the HITRAN compilation e.g. those in the GEISA compilation. Isotope identifiers also agree as far as possible with HITRAN identifiers. Similar (1,2,3...n) identifiers have been defined for additional gases.

Makedb also converts the sequential files output by Select and Cp_lines into direct access databases.

### 7.1.2 Scan

This program opens a line database specified by an input key file name and lists the line data records directly to screen. The program prompts for a starting wavenumber and then lists all transitions from that point in the storage format until told to abort.

### 7.1.3 Check_index

Check_index opens a line database specified by an input key file name and lists the index file records directly to screen. The program prompts for a starting wavenumber and then lists all transitions from that point in the storage format until told to abort.

### 7.1.4 Li_lines

This program opens a line database specified by an input key file name and lists selected line data directly to screen. The line data is read in by the subroutine lines.f which in turn reads in line data records with the subroutine rdline.f. To avoid numerical underflow when passing line strengths between these subroutines, the strengths are multiplied by the factor 1047 in rdline.f. Before listing the strengths therefore, Li_lines multiplies the returned strengths by 10-47 so that the strengths are listed in units of cm-1/(molecule cm-2) as defined in the line databases (see section 2.1). The program prompts for the required gases and the wavenumber range and then lists main line data parameters for these gases.

### 7.1.5 Pl_lines

Pl_lines plots out line details for a particular spectral region ASCII format, suitable for display using the IDL routine pl_alines.pro. The program prompts for line data key, gas identifiers, wavelength range and plotting details and then plots out line strengths versus wavenumber. The same scaling and de-scaling of the line strengths by the factor 1047 is applied as in Li_lines.

### 7.1.6 Select

This a line data manipulation/editing program which opens a line database created by Makedb (e.g. HITRAN or GEISA) specified by an input key file name and copies a specified subset of the line database to a new sequential ASCII file. This file may then be modified and perhaps merged with new spectral lines before being turned back into a new direct access database using Makedb. All lines or only selected lines between given wavenumber limits are copied. Selection can be performed by gas ID, isotope, and line strength limit. The strength limit for a particular gas or isotope is set so that the sum of strengths of omitted lines is less than n% of the total sum of strengths. This criteria is calculated by finding the number of lines in each decade of strength. Note that the strength limit can be set independently for each isotope or can be set for all of the isotopes to be included. In the latter case terrestrial isotopic abundances are assumed, i.e the strengths are used uncorrected. The logic behind this is that if the isotopic abundance may vary you want to treat them independently anyway. The line data may also be modified to have, for example, different line widths for comparison with other programs or to check details of the line data itself.

Example input sequence:

Enter wavenumber limits :1400,1450

Enter bin size :10

Enter error limit [%] :0

Enter new file name :test

set self broadened width to air broadened? :n

multiply H2O air broadened width by 1.3? :n

replace H2O air b. by delaye CO2-broadened data? :n

generate linedata using PMR2 assumptions? :n

generate linedata using Wedads assumptions? :n

generate linedata using Brunos assumptions? :n

generate linedata using all PMIRR assumptions? :n

H2-He broaden PH3 for Jupiter? :n

--------------------------------------------------

enter gas details

enter gas id and isotope [0=all isotopes]

ALL includes all gases, GO starts execution,

? gives summary

ID ISO ? :1,0

ID ISO ? :GO

As can be seen, the new sequential file is called 'test' and contains linedata between 1400 and 1450 cm-1 for all isotopes of water vapour only. The program may then perform a number of modifications to the line data which are historical in nature. The first three possible operations are self-explanatory, the data for the third operation coming from [R8]. It should also be noted that the third option replaces the transmission probability column with the temperature dependence of the self-broadening since this column is not normally used by Radtrans (This data is not used in version 1.1). The last three operations are more obscure.

PMR2 Assumptions - This operation generates linedata equivalent to that used by PMR2, a non-overlapping line model developed by J.T.Schofield. Modifications are:

H2O	Multiply air broadening coefficient by 1/1.013246

Set self-broadening to 6.5 times air broadening

Set temperature dependancies to 0.5.

Replace transition probability column with the temperature dependence of the self-broadening

CO2	Set the air and self broadened widths to those calculated by Yamamoto [R9] multiplied by the empirical factor 1.1/1.013246.

Set the temperature dependancies to those calculated by Yamamoto.

Replace transition probability column with the temperature dependence of the self-broadening

Wedad Assumptions - This operation generates linedata equivalent to that used by Wedad Adbou at JPL for some of the PMIRR intercomparison tests. Modifications are:

H2O	Multiply air broadening coefficient by 1/1.013246

Set self-broadening to 6.5 times air broadening

Set temperature dependancies to 0.5.

Replace transition probability column with the temperature dependence of the self-broadening

CO2	If the self broadened width is listed as zero, set it to that calculated by Yamamoto [R9] multiplied by 1/1.013246.

Set the temperature dependancies to those calculated by Yamamoto.

Replace transition probability column with the temperature dependence of the self-broadening

Bruno Assumptions – Various modifications to linewidths etc. for CIRS Jovian radiative transfer tests.

PMIRR Assumptions - These are the assumptions that would have been used to generate the final transmittance tables for PMIRR. Modifications are:

H2O	Replace air broadened width and temperature dependence of water with Delaye calculated CO2 broadened value

Replace transition probability column by temperature dependence of self broadening calculated from Delaye

If self broadening is set to zero, substitute Delaye calculated value.

CO2	Leave air broadened CO2 unchanged

If self broadening is listed as zero, set it to that calculated by Yamamoto/1.013426

Replace air broadened temperature dependence with that calculated by Yamamoto

Replace transition probability column by temperature dependence of self broadening calculated from Yamamoto

H2-He broaden PH3 for Jupiter – self-explanatory.

### 7.1.7 Selecttemp

Identical to Select, except that the line strength selections are made at a temperature of the user’s choice. This is because at different temperatures, different lines become more important depending on their lower state energies. This program also requires the user to specify whether IPTF=0 and the default partition functions (defined by the gasinfo file) are to be used or IPTF=1, in which case the partition functions of those gases listed in the extra partition function file are to be used instead.

### 7.1.8 Selecttemploop

Identical to Selecttemp, except that the several line selections are made for a range of different temperatures defined by the user. Again, the variable IPTF must be defined by the user.

### 7.1.9 Cp_lines

Cp_lines opens a line database created by Makedb (eg HITRAN or GEISA) specified by an input key file name and copies a specified subset of the line database to a new sequential ASCII file just like Select. Unlike Select however, it copies all lines between given wavenumber limits rather than selecting them by gas, isotope and strength.

### 7.1.10 Summary

This program opens a line database specified by an input key file name and prompts for a wavelength range and bin width. Summary then producues an ASCII output file called "summary.out" which summaries the content of the datebase for each gas for each bin in the specified range. The first few lines of a typical summary file are listed below:

database: hitran92

from DB file: /vol/local/linedata/hitran92.asc

as defined in key: hitran92.key

--------------------------------------------------------

BIN:  1     1400.000 -  1410.000 cm-1

id iso    smin               smax        nlines

H2O      1   1  0.47000E-26  0.27800E-19     29

H2O      1   2  0.28800E-25  0.55200E-22     11

H2O      1   3  0.52400E-26  0.10300E-22     11

H2O      1   4  0.10370E-26  0.96180E-23     29

total of      80  H2O     lines

id iso    smin               smax        nlines

CO2      2   3  0.94910E-26  0.70040E-25     40

CO2      2   4  0.50400E-26  0.19980E-25     13

total of      53  CO2     lines

id iso    smin               smax        nlines

O3       3   1  0.10200E-24  0.29100E-23    280

total of     280  O3      lines

### 7.1.11 Merge

This program takes two sequential access line database files and then merges them together in order of wavenumber. This program is very useful for adding extra lines to existing line databases.

### 7.1.12 Line data Strengths and Absorber Amounts Units

This area causes a lot of confusion! When Radtrans reads in line data from one of the databases set up by Makedb, they do so using the subroutine lines.f which in turn uses the subroutine rdline.f. The units of line strength in the line databases are the standard ones of cm-1/ (molecule cm-2) and as such have rather small exponents. In order to avoid numerical underflow when passing small strengths between the subroutines, rdline.f multiplies the strengths by the factor 1047. Thus when the final calculation routines get hold of the strengths, they must be converted back to their correct units using the factor 10-47. This number is too small to be applied in one go using real*4 numbers and so is applied in two stages. The factor 10-27 is applied directly and the factor 10-20 is applied to the absorber amounts which are defined in the units of (molecule cm-2).

### 7.1.13 Partition Functions

The line strengths defined in databases such as HITRAN are for a specific temperature only, T0 = 296K, and must first be scaled to be used at any other temperature. The line strength at temperature, T, is related to that at the reference temperature, T0, by the equation:

ST=ST0×fZ×eαE×fS

where:

fZ is a term to scale the strengths by the relative partition functions at the two temperatures fZ=ZT0ZT where Z(T) is the total partition function at temperature T, and is equal to Zrot(T) Zvib(T). At the usual temperatures found in the solar system atmospheres, Z is dominated by the rotational partition function Z(T) ≈ Zrot(T) = Tn, where n=1 for linear molecules and 1.5 for non-linear molecules. At higher temperatures, the vibrational partition function can come into play.

E is the lower state energy of the line  (expressed in cm-1).

 is a temperature factor α=1.439(T-T0)/(T0T)

fS is a stimulated emission term given by fS=1-exp⁡(-1.439ν/T)1-exp⁡(-1.439ν/T0), where ν is the wavenumber of the line.

For standard line databases covering the temperature ranges found in the Solar System atmospheres, the partition function is fitted with a cubic polynomial and the four coefficients listed in the gas_info file for each isotope of each gas recognised by the software, in addition to their relative abundances. At the calculation temperature, T, the total partition function is then calculated as:

ZT=c0+c1T+c2T2+c3T3

where c0-3 are the listed coefficients. This system works well over limited temperature ranges, but can quickly diverge at temperatures outside the normal range, even leading to negative partition functions. Modelling extrasolar planets, whose temperatures can far exceed those for which the polynomials are valid can be extremely problematic and in this case, Radtrans allows the user to override the partition functions calculated with the polynomials for specific gases by values interpolated from a table supplied by the user in …/raddata/partfextra.dat. The format of this file is:

# Any number of lines of header preceded by a #

NPARTEXTRA		! Number of gases for which data exist in table

IDEXTRA(1),ISOEXTRA(1)	! ID, ISO of gases listed. ISOEXTRA(I)=0 is allowed.

. . .

IDEXTRA(NPARTEXTRA),ISOEXTRA(NPARTEXTRA)

# Any number of lines of header preceded by a #

NTAB(1)			! Number of temperatures for 1st gas in list

TEMP(1,1) PARTF(1,1)

. . .				! Temperatures and partition functions.

TEMP(1,NTAB(1)),PARTF(1,NTAB(1))

# Any number of lines of header preceded by a #

NTAB(2)			! Number of temperatures for 2nd gas in list

TEMP(2,1) PARTF(2,1)

. . .				! Temperatures and partition functions.

TEMP(2,NTAB(2)),PARTF(2,NTAB(2))

. . . and so on.

Please note that PARTF(I,J) = fZ = Z(T0)/Z(T).

## 7.2 Band Data

Radtrans contains some spectral models which use band data instead of line data. When these routines are used, the spectral data must be read in from a band parameter file which has a ‘.ban’ suffix. This band data file may be generated either by statistical analysis of line data in the HITRAN or GEISA databases or calculated from low resolution laboratory experiments. For every ‘.ban’ file there must also be a .key file which contains the location of the ‘.ban’ file and the gas identifier files

A typical ‘.ban’ file has the following format:

------------------------------------------------------------------------------

Original file name : 5jupiter5.ban

Originator : Pat Irwin

Date : 25/10/94

Data sources :

Gas :   6  1 HITRAN92

Gas :  11  0 "

Gas :   6  3 "

Gas :  28  0 "

Gas :  33  0 "

Gas :   1  0 "

Corrected line treatment in Makeband

*******************************************************

VMIN =  1800.00

DELV =     5.00

FWHM =     5.00

NPOINT =    61

NGAS =   6

Gas :   6  1

Gas :  11  0

Gas :   6  3

Gas :  28  0

Gas :  33  0

Gas :   1  0

------------------------------------------------------------------------------

Band data for gas :   6  1

V0       dV       Kv(T0)    delta/AD0      y0           El       SFB

1800.00    5.00 0.52856E-03 0.88439E+03 0.26271E+03 0.14892E+04 0.13373E+01

1805.00    5.00 0.11291E-02 0.91134E+03 0.27025E+03 0.14911E+04 0.13193E+01

1810.00    5.00 0.12633E-03 0.11360E+04 0.45271E+03 0.16362E+04 0.13533E+01

1815.00    5.00 0.38263E-03 0.12947E+04 0.37365E+03 0.16080E+04 0.13393E+01

1820.00    5.00 0.89474E-03 0.93499E+03 0.25684E+03 0.15506E+04 0.13320E+01

The first half of the header contains information on the data sources, who generated the file and when. The second half contains the spectral details of the file and the number of gases contained.

The data itself contains the following columns for each spectral bin defined for each gas:

El	SFB

where:

is the central wavenumber of the bin

is the width of the bin in wavenumbers

is 1020  the mean absorption coefficient at  = 296K (units of absorption are (molecule cm-2)-1)

is the mean line spacing  over the mean Doppler width coefficient .

is the mean self-broadened Lorentz width at ,  (= 1 atm) over the mean doppler width coefficient.

El is the mean lower state energy (cm-1)

SFB is the mean self-to-foreign broadening ratio.

is the Doppler width coefficient for the bin which depends only on the mean wavenumber of the bin and the molecular mass. The Doppler width at temperature T is then found from:

A typical ‘.key’ file has the following format:

5jupiter5.ban

/home/plan/linedata/gas_info.dat

### 7.2.1 Makeband

The data in the ‘.ban’ files may come either from laboratory experiments or from analysis of line databases. The program which performs this latter function is Makeband which also generates a ‘.key’ file. The band parameters defined above are derived from the linedata as follows:

From [R4], the mean absorption in a spectral interval is given by:

where	 is the sum of the line strengths and the mean line spacing is defined as .

The mean Doppler width coefficient depends on the mean wavenumber and absorber molecular weight (g) only through the equation:

In Section 5 we saw that the Goody and Malkmus distributions have different definitions of  with the Malkmus value being 4 times smaller. Using the Goody definition the mean self-broadened half-width at T0, P0 may be calculated to be:

where . When using this table to calculate Malkmus bands, the value of  must thus first be divided by 4.

The mean self/foreign broadening ratio and mean lower state energy are simply the mass weighted means:

Makeband is a user-interactive program which is fairly self-explanatory. The input session used to create the band parameter file in the example is shown below. N.B. Gas Ids are Radtrans IDs.

Enter name of lbl key file :/home/oxpln3/plan/linedata/hitran92

38 gases recognised by software

32 gases in database

Enter wavenumber range and spacing.

Enter Vmin, Vmax, DelV, FWHM :1800,2100,5,5

Enter number of gases :6

Gas :            1

Enter Gas ID and ISO :6 1

Gas :            2

Enter Gas ID and ISO :11 0

Gas :            3

Enter Gas ID and ISO :6 3

Gas :            4

Enter Gas ID and ISO :33 0

Gas :            5

Enter Gas ID and ISO :28 0

Gas :            6

Enter Gas ID and ISO :1 0

Enter name of output file :5jupiter5

%writing output

Enter Originator :Pat Irwin

Enter Date :15/10/94

Enter source of data for gas   6  1

HITRAN92

Enter source of data for gas   11 0

"

Enter source of data for gas   6  3

"

Enter source of data for gas   33 0

"

Enter source of data for gas   28 0

"

Enter source of data for gas   1  0

"

Enter any additional header information

Corrected line treatment in Makeband

%calculation complete

In this example the spacing of the points and the bin width is the same so that the bins do not overlap. Hence the resultant spectra are under-sampled. Nyquist sampling (at least!) is usually recommended. DELV and FWHM are independent of each other and can be set at will.

### 7.2.2 Convert_data

The routines written by Kim Strong for fitting band parameters to laboratory measurements produce an output which is similar but not identical to the .ban format used by Radtrans. The output from Kim’s routines can be converted using Convert_data.

Unfortunately, Kim’s files do not specify which parameter is which so that the correspondence between this output and the ‘.ban’ format has to be hard-coded into Convert_data. In particular, Convert_data is currently set up to convert the fits made of Simon Calcutt’s NH3 data by Kam Sihra which do not include an SFB parameter. Thus the code will have to be modified to cope with any subsequent changes to the output from Kim’s routines.

## 7.3 Correlated-k data

Radtrans may also operate with pre-calculated correlated-k tables. These are created by the program Calc_ntable (line data) and Band_ktable (band data) and may be inspected using the program Read_table.

## 7.4 Gas Continua and Collision Induced Absorption (CIA)

Collision-induced absorption by Hydrogen-Helium has now been incorporated into Radtrans [R12,13]. Additional CIA coefficients for N2, CH4 have also been incorporated. You must tell Radtrans which CIA table to use (which are all stored in raddata/ directory, by including a .cia file, whose format is described in the Nemesis manual.

Other sources of continuum opacity such as the H2O continuum absorption routines used in GENLN2, together with NIR methane absorption coefficients from Karkoschka (1994) and Fink et al. (1997) are built into the subroutine gascon.f, which should provide a means of easy expansion. To turn these continuum absorptions on or off, you need to define the appropriate flags in the .fla file, whose format is described in the Nemesis manual.

# 8. Scattering

Radtrans allows the calculation of thermal emission and reflected sunshine in a multiply-scattering atmosphere through the use of the Matrix Operator Method (also known as Doubling and Adding). This section has been compiled from previous technical reports [R16, R17] to describe Radtrans scattering calculations in one place and also to emphasise how it fits within Radtrans. [R15] gives a general introduction to scattering in planetary atmospheres.

## 8.1 Phase Functions and Normalisation

All particles scatter radiation as a function of the scattering angle  called the Phase Function. For simple scattering cases such as Rayleigh scattering this is a simple function of cos. In Radtrans, the phase function is calculated by the routine phase1.f. To conserve energy in the model we need some sort of normalisation. The one chosen for Radtrans is that:

or

where = cos. Now in Radtrans, the phase function at a defined incident and emission zenith angle is expressed as a Fourier series and is calculated by calc_pmat6.f :

In this case the normalisation requirement reduces to just:

Of course the Radtrans scattering code uses numerical techniques to evaluate the radiative transfer and in particular the integration over zenith angles above is simplified by using Lobatto quadrature to a simple weighted sum. In addition, instead of writing a total integral between negative and positive values of , the phase function is split into two parts, one where the incident and scattered radiation are in the positive  direction called  and one where the incident and scattered radiation are in opposite directions called . By symmetry arguments,  and . The normalisation requirements then become:

For any but the simplest phase functions, inherent inaccuracies in the zenith quadrature scheme reveal themselves in the above equation, and if uncorrected lead to highly unphysical results such as negative reflectivities for thick clouds. Hence renormalisation is almost always required and then one most commonly used is that of [R18]. Here, the reverse scattering matrix is left unaltered (since for most cases the inaccuracies arise in the forward part) and instead for forward scattering matrix is modified to ensure that for illumination angle j, the sum of the reflected and transmitted intensities adds up to one, i.e. tj+rj=1 where:

,

In the Hansen scheme [R18], each element of the forward matrix for each Fourier component is modified by the correction matrix fij which is found iteratively for the zeroth order Fourier component and ensures that :

(8.7)

### 8.1.1 Phase functions used in Radtrans

The simplest quasi-realistic phase function available is the Henyey-Greenstein phase function:

where  is the scattering angle (0 = forward), g is the asymmetry factor (1=forward, 0=isotropic and -1 = backward), and the factor of 4 ensures the correct normalisation:

This function is easy to use and has only one defining parameter but is actually not always very practical since most scatterers have both a forward- and back-scattering peak. Hence a more general purpose function is a combined Henyey-Greenstein function:

where f is some fraction between 0 and 1 and g1, g2 are the asymmetry factors of the two contributing functions. This is the form of the H-G phase function used by Radtrans. To be specific, in Radtrans:

This function can quite accurately reproduce most phase function curves.

## 8.2 Fourier Decomposition

The routine phase1.f calculates the phase function at required scattering angles  and this function is Fourier analysed to determine the Fourier coefficients by the routine rephase.f over the range 0 <  <  in the normal way. The phase function can then of course be written as:

(1)

Now in calc_pmat6.f, we want to calculate the azimuth angle Fourier components of the phase function at fixed incident and scattered zenith angles, i.e:

These are both equal to Eq.1 if we use geometry to calculate the phase angle  from zenith and azimuth angles:

The required Fourier components br and cr are then found from the calculated phase angle components an from the matrix equations: b = A+a and c = A-a where the elements of the A matrices are:

and these integrations are performed once by ftable.f using a simple trapezium rule code and stored.

## 8.3 Doubling and adding

Consider two homogeneous layers in an atmosphere. For any incident, emission and azimuth angle (i, j,) we can define coefficients of reflection and transmission as well as a value for the source function, giving the radiance emanating from within the layer due to thermal emission. Taking the positive  direction to be downwards, and for now taking

(2)

where the extra  factor is due to the illumination angle geometry. similarly

First of all, we can see that Fourier decomposition in azimuth angle works! Consider the 1st term on the RHS of Eq. 2. Writing the radiance and transmission as Fourier series we get:

where the integrals over the cross-terms average out leaving just the diagonal elements, forming the required Fourier series. In practice of course, the integral over zenith angle is replaced by the Lobatto quadrature scheme:

and thus the integral equations are replaced by matrix equations:

which may be rearranged to give:

This is what the subroutine add.f does where the subscripts become:

which looks a lot simpler!

The last complication is how do you start off? We need an initial reflection and transmission matrix for one layer from which we’ll calculate everything else. The scheme used is to start with a very optically thin layer where then:

where

,

The various elements are not so obscure as they at first seem! The product of  gives the optical depth at each zenith angle. The (1+or) term deals with the integration over azimuth angle factors (see Eq. 14) and the C matrix contains the weights required for this matrix version of an integral equation.

Consider the case for isotropic scattering where p=1/4 for the zeroth order only. Then:

The subroutine double1.f calculates the required matrices for a homogeneous layer by first defining a very thin layer as above and then doubling up the thickness N times to get the resultant matrices. In this case N is 16.

### 8.3.1 Quadrature details

In the scattering code it is assumed that for light incident at  and leaving at , where  relates to zenith angles and  to azimuth angles, the phase function may be expanded in the Fourier series:

With such an expansion, the coefficients may be found in the usual way:

for l=0 and for l>0 :

Hence the first check for a sufficiently precise quadrature scheme is that once N coefficients have been calculated, that they adequately describe the function to be fitted to within a prescribed error limit. Using this expansion of the phase matrix, it is assumed that a similar expansion applies to the final computed radiances also [R19]. Thus the PM coefficients calculated above may be used in N+1 independent radiative transfer equations and the final radiances calculated at the end by combining all the components.

The second quadrature takes place over zenith angle. Checking the fit here is more problematic but a good place to start is to ensure that the integral of the phase function over solid angle is close to unity. i.e.:

i.e.:

As can be seen, all integrals for l > 0 are identically zero and hence the normalisation check refers to the 0th order only. In practice, the scattering code splits up the phase function into forward and reverse components and, using Guassian-Lobatto quadrature which allows computation of radiances at =±1, integrates as:

,

Clearly we want to ensure that tj+rj=1 and this is exactly what the renormalisation scheme of Hansen [R18] does, where the forward part of the phase function is iteratively modified until . Such an approach is sensible for  highly forward scattering particles where the main quadrature error appears in the forward part but is maybe of dubious quality for particles with both sharp forward and back scattering peaks. Whatever the quality of the assumption however, it is probably safe to assume that there are insufficient quadrature points if j is greater than some prescribed value (say 2%) for any value of j and this can be used as a check for the zenith integration.

## 8.4 Miscellaneous Calculation Details

Final points about averaging and so on are as follows. For a Lambertian reflecting layer, the matrix is defined as, for zeroth order Fourier component only as:

And for incident sunlight which is clearly an irradiance from a specific zenith and azimuth angle, the incident radiance vector is defined as:

where the appearance of the weight w2 ensures correct treatment in the zenith quadrature of a delta function.

## 8.5 Calculating scattering properties

If a scattering path is defined in the ‘.pat’ path file, then for the Radtrans program to run, further details of the scattering calculation must be supplied. Extinction cross-sections and single scattering albedos are already supplied in the ‘.xsc’ file discussed previously. However Radtrans also needs to know how to calculate the phase function of the aerosols and how to integrate over the solid angles. All these details are contained in the ‘.sca’ file which is discussed more fully below. For simple cases like dipole and isotropic scattering, the calculation of the phase function may be performed in real time. However for more complicated cases like Mie scattering (which is the analytical solution to the case of scattering of plane electromagnetic waves by dielectric spheres) the calculation must be done off-line using the program Makephase. It should be noted that Makephase also writes the extinction cross-section and single scattering albedo ‘.xsc’ file.

### 8.5.1 Calculating scattering properties with Makephase

Makephase is an interactive program which prompts the user on how to set up the Mie calculation. When the program is invoked, the user is given the following calculation options (N.B. the term ‘mode’ below refers to a particular particle type, i.e. composition, radius, size distribution):

Enter number of aerosol modes :

2

enter wavenumber range:  (start / end / delta)

1900,2100,200

Enter phase function calculation ID:           1

1) Mie Scattering. Standard Gamma Distribution

2) Mie Scattering. Log-Normal distribution

3) Mie Scattering. Single particle size

4) Isotropic scattering

5) Henyey-Greenstein scattering

6) Dipole scattering

As can be seen the user must first define the number of aerosol types and the required wavenumber grid. The user must then select the required model following which certain other constants must be supplied:

1) Mie Scattering. Standard Gamma Distribution

Here the aerosols are assumed to have the following size distribution [R11]:

Here a is the effective mean radius of the distribution and b is the effective variance.

The user must thus supply first both a and b.

The user must then either enter the real and imaginary parts of the refractive index of the aerosol or specify a refractive index table.

The user may then let Makephase select the integration range over the radius distribution to determine the mean phase function or may set it manually

The extinction cross-section and single scattering albedo are calculated automatically and written to a ‘.xsc’ file.

2) Mie Scattering. Log-Normal distribution

Here the aerosols are assumed to have the following size distribution:

where rg is the mean radius of the particle distribution and g may be thought of as the mean value of ln(r)/ln(rg) .

Hence the user must supply rg (in microns) and g.

After that the user must then again supply the refractive index and the preferred integration range (if required).

The extinction cross-section and single scattering albedo are calculated automatically and written to a ‘.xsc’ file.

3) Mie Scattering. Single particle size

Here the user is simply required to supply the mean particle radius followed by the refractive index.

The extinction cross-section and single scattering albedo are calculated automatically and written to a .xsc file.

4) Isotropic scattering

Here the phase function is set to be uniform in all directions. Makephase prompts the user for the extinction cross-section and single scattering albedo at each wavelength

5) Henyey-Greenstein scattering

The Henyey-Greenstein phase function (a useful parameterised approximation to real phase functions) was described earlier and is defined as:

where the constants c1, c2 and c3 are supplied by the user, and  is the scattering angle.

Makephase again prompts for the extinction cross section and single scattering albedo.

6) Dipole scattering

Here the user must supply the mean particle radius (microns) and the complex refractive index. The phase function is then calculated as:

Where x is the size parameter 2r/ , nc is the complex refractive index and Qsca is the scattering efficiency factor which is the ratio of the scattering x-section to the geometrical x-section. By conservation of energy:

Where Aext is the extinction cross section and  is the single scattering albedo.

In all cases the calculated phase functions are written to the files PHASENN.DAT where the two-digit number NN corresponds to the number of the mode.

### 8.5.2 Format of ‘.sca’ file.

If a scattering path is defined in the ‘.pat’ file, then a scattering ‘.sca’ file must exist which contains additional details of the calculation. This file also has the same name as the driver and has the following format:

LOBATTO.DAT generated by Testlob    18/11/94

21

3.784971655960363E-002  7.566327910711329E-002

0.113332349888654       7.522988661327083E-002

0.188165825654151       7.436558405320763E-002

0.261921504390139       7.307532206213684E-002

0.334176927804947       7.136648893356323E-002

0.404518200290651       6.924887923879364E-002

0.472542436198248       6.673461588649052E-002

0.537859990923905       6.383809986245448E-002

0.600096702575684       6.057592108845711E-002

0.658896148204803       5.696651339530945E-002

0.713921568917289       5.303148882011104E-002

0.764857292175293       4.878216609358788E-002

0.811413731158588       4.432398443333577E-002

0.853316624328711       3.944781980769774E-002

0.890346703886177       3.485291182131400E-002

0.922246795926299       2.863839588550722E-002

0.948912561286221       2.528072619188358E-002

0.970082573462594       1.693047310774074E-002

0.985752649473397       1.542868909873032E-002

0.995737073151015       6.230266387428623E-003

1.00000000000000       1.161440185830430E-003

1                                     ! isol

5.2                                  ! dist

1                                     ! lowbc

1.0                                  ! galb

42.0, 0.0                         ! sol_ang,emiss_ang

0.0                                  ! aphi

0                                     ! nf (azimuth orders)

3                                     ! ncont

1                                     ! iscat(1)

1                                     ! norm(1)

0                                     ! ncons(1)

1                                     ! iscat(2)

1                                     ! norm(2)

0                                     ! ncons(2)

1                                     ! iscat(3)

1                                     ! norm(3)

0                                     ! ncons(3)

The first line contains the header and the second line the number of points N to use in the zenith angle quadrature. The next N lines contain the ordinates of cos(theta) and the weights to use in the Gaussian-Lobatto quadrature. The remaining parameters are defined as follows:

isol	integer	Sunlight switch.

0=night

1=day

dist	real	Distance of planet from sun in AU

lowbc	integer	Lower boundary condition identifier:

0 = thermally emitting boundary

1 = Lambertian reflecting boundary

galb	real	Albedo of lower boundary

sol_ang	real	Sunlight incident zenith angle

emiss_ang	real	Viewing, or emission, zenith angle

aphi	real	Azimuth angle between incident and viewing directions

nf	integer	Number of orders in azimuth fourier expansion. 0 = azimuthally symmetric

ncont	integer	Number of scattering modes

iscat	integer	Phase function identifier

0 = dipole

1 = isotropic scattering

2 =Henyey-Greenstein -

(3 coefficients read into array CONS below)

3 =Legendre polynomial

(ICONS (<= 10) coefficients in read into array CONS below)

4 = Phase function read in from disk file

5 = Henyey-Greenstein pretabulated phase functions

norm	integer	Normalization identifier:

0 = none

1 = Hansen

2 = Barkstrom

ncons	integer	Number of constants defined for Phase function

cons	real(10)	List of ncons constants

Note: if iscat > 3 then ncons and cons are replaced by the name of the phase function file (created by Makephase) for the particular dust mode such that the end of the file looks like this:

1	! isol

5.2	! dist

1	! lowbc

1.0	! galb

42.0, 0.0	! sol_ang,emiss_ang

0.0	! aphi

0	! nf (azimuth orders)

3	! ncont

4	! iscat(1)

1	! norm(1)

PHASE1.DAT	! phase(1)

1	! iscat(2)

1	! norm(2)

PHASE2.DAT	! phase(2)

1	! iscat(3)

1	! norm(3)

PHASE3.DAT	! phase(3)

# Appendix A. Radtrans gas ID codes

Since HITRAN and GEISA use slightly different gas identifiers, Radtrans uses its own gas identification system, based on HITRAN, so that both data formats can be transcribed. The gas identification system used is shown in Table A.1. The local gas identifier agrees with HITRAN IDs as far as possible (Jan 1988) with extra IDs for gases not in the HITRAN compilation e.g. those in the GEISA compilation. Isotope identifiers also agree as far as possible with HITRAN identifiers. Similar (1,2,3...n) identifiers have been defined for additional gases. For example, ID = 1, ISO =1 refers to the first isotope of H2O. ID = 6, ISO = 3 is CH3D. ID = 6, ISO = 0, means use all isotopes of CH4 with relative isotope abundances equal to their terrestrial values.

Table A.1 Gases Defined for RADTRAN

| Radtran ID | Gas | Hitran92 ID | Hitran08 ID | Geisa92 ID | Geisa97 ID | Geisa03 ID | Number of Isotopes | Isotopes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H2O | 1 |  | 1 | 1 |  | 5 | 161,181,171,162,182 |
| 2 | CO2 | 2 |  | 2 | 2 |  | 9 | 626,636,628,627,638,637,828,728,838 |
| 3 | O3 | 3 |  | 3 | 3 |  | 5 | 666,668,686,667,676 |
| 4 | N2O | 4 |  | 4 | 4 |  | 5 | 446,456,546,448,447 |
| 5 | CO | 5 |  | 5 | 5 |  | 6 | 26,36,28,27,38,37 |
| 6 | CH4 | 6 |  | 6 | 6 |  | 3 | 211,311,212 |
| 7 | O2 | 7 |  | 7 | 7 |  | 3 | 66,68,67 |
| 8 | NO | 8 |  | 8 | 8 |  | 3 | 46,56,48 |
| 9 | SO2 | 9 |  | 9 | 9 |  | 2 | 626,646 |
| 10 | NO2 | 10 |  | 10 | 10 |  | 1 | 646 |
| 11 | NH3 | 11 |  | 11 | 11 |  | 2 | 411,511 |
| 12 | HNO3 | 12 |  | 13 | 13 |  | 1 | 146 |
| 13 | OH | 13 |  | 14 | 14 |  | 3 | 61,81,62 |
| 14 | HF | 14 |  | 15 | 15 |  | 1 | 19 |
| 15 | HCL | 15 |  | 16 | 16 |  | 2 | 15,17 |
| 16 | HBr | 16 |  | 17 | 17 |  | 2 | 19,11 |

| Radtran ID | Gas | Hitran92 ID | Hitran08 ID | Geisa92 ID | Geisa97 ID | Geisa03 ID | Number of Isotopes | Isotopes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17 | HI | 17 |  | 18 | 18 |  | 1 | 17 |
| 18 | ClO | 18 |  | 19 | 19 |  | 2 | 56,76 |
| 19 | OCS | 19 |  | 20 | 20 |  | 7 | 622,624,632,822,623,634,722 |
| 20 | H2CO | 20 |  | 21 | 21 |  | 3 | 126,136,128 |
| 21 | HOCl | 21 |  | 32 | 32 |  | 2 | 165,167 |
| 22 | N2 | 22 |  | 33 | 33 |  | 1 | 44 |
| 23 | HCN | 23 |  | 27 | 27 |  | 3 | 124,134,125 |
| 24 | CH3Cl | 24 |  | 34 | 34 |  | 2 | 215,217 |
| 25 | H2O2 | 25 |  | 35 | 35 |  | 1 | 166 |
| 26 | C2H2 | 26 |  | 24 | 24 |  | 2 | 221,231 |
| 27 | C2H6 | 27 |  | 22 | 22 |  | 2 | 226,236 |
| 28 | PH3 | 28 |  | 12 | 12 |  | 1 | 131 |
| 29 | C2N2 | n/a |  | 29 | 29 |  | 1 | 224 |
| 30 | C4H2 | n/a |  | 30 | 30 |  | 1 | 211 |
| 31 | HC3N | n/a |  | 31 | 31 |  | 1 | 124 |
| 32 | C2H4 | n/a |  | 25 | 25 |  | 2 | 211,311 |
| 33 | GeH4 | n/a |  | 26 | 26 |  | 1 | 411 |
| 34 | C3H8 | n/a |  | 28 | 28 |  | 1 | 211 |
| 35 | HCOOH | 32 |  | 37 | 37 |  | 1 | 261 |
| 36 | H2S | 31 |  | 36 | 36 |  | 3 | 121,131,141 |
| 37 | COF2 | 29 |  | 38 | 38 |  | 1 | 269 |

| Radtran ID | Gas | Hitran92 ID | Hitran08 ID | Geisa92 ID | Geisa97 ID | Geisa03 ID | Number of Isotopes | Isotopes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38 | SF6 | 30 |  | 39 | 39 |  | 1 | 29 |
| 39 | H2 | n/a |  | n/a | n/a |  | 2 | 11,12 |
| 40 | He | n/a |  | n/a | n/a |  | 1 | 4 |
| 41 | AsH3 | n/a |  | n/a | n/a |  | 1 | 181 |
| 42 | C3H4 | n/a |  | 40 | 40 |  | 1 | 211 |
| 43 | ClONO2 | n/a |  | n/a | 42 |  | 2 | 564,764 |
| 44 | HO2 | n/a |  | n/a | 41 |  | 1 | 166 |
| 45 | O |  |  |  |  |  | 1 | 6 |
| 46 | NO+ |  |  |  |  |  | 1 | 4 |
| 47 | CH3OH |  |  |  |  |  | 1 | 211 |
| 48 | H2 |  |  |  |  |  | 2 | 11,12 |
| 49 | C6H6 |  |  |  |  |  | 1 | 221 |
| 50 | CH3CN |  |  |  |  |  | 1 | 241 |
| 51 | CH2CCH2 |  |  |  |  |  | 1 | 221 |
| 52 | C4N2 |  |  |  |  |  | 1 | 224 |